| DCUSA Consultation |  | At what stage is this document in the process? |
| --- | --- | --- |
| DCP 313 Eligibility Criteria for EDCM Generation Credits Raised on the 10 October 2017 as a Standard Change |  | 01 – Change Proposal. 02 – Consultation. 03 – Change Report. 04 – Change Declaration. |
| Purpose of Change Proposal: The intent of this Change Proposal is to improve transparency of the eligibility criteria for EDCM generators to receive super red credits, and to improve consistency in the application thereof. |  |  |
|  | The Workgroup recommends that this Change Proposal should: proceed to Consultation ## Parties are invited to consider the questions set in section 9 and submit comments using the form attached as Attachment 1 to dcusa@electralink.co.uk by XX XXX XX DCP 313 has been designated as a Part 1 Matter and a standard change. The Working Group will consider the consultation responses and determine the appropriate next steps for the progression of the Change Proposal (CP). |  |
|  | Impacted Parties: DNOs, Generators and Suppliers |  |
|  | Impacted Clauses: Schedule 17 – EHV Charging Methodology (FCP Model); and Schedule 18 - EHV Charging Methodology (LRIC Model). |  |

| Contents 1. Summary 3 2 Governance 4 3 Why Change? 4 4 Working Group Assessment 6 5 Legal Text 11 6 Relevant Objectives 13 7 Impacts & Other Considerations 14 8 Implementation 16 9 Consultation Questions 16 Timetable The timetable for the progression of the CP is as follows: #### Change Proposal timetable Change Proposal timetable:. #### Activity; #### Date. Initial Assessment Report Approved by Panel; 11 October 2017. First Consultation issued to Parties; 23 February 2018. Second Consultation issued to Parties; XX XXX XX. Change Report issued to Panel; 12 December 2018. Change Report issued for Voting; 21 December 2018. Party Voting Ends; 16 January 2019. Change Declaration Issued to Parties; 18 January 2019. Authority Decision; 12 February 2019. Implementation; 01 April 2021. | Any questions? |
| --- | --- |
|  | Contact: Code Administrator |
|  | DCUSA@electralink.co.uk |
|  | 02074323000 |
|  | Proposer: Andrew Enzor |
|  | andrew.enzor@northernpowergrid.com |
|  | 07834 618994 |

- Summary

#### What?

## The Distribution Connection and Use of System Agreement (DCUSA) is a multi-party contract between electricity distributors, electricity suppliers and large generators. Parties to the DCUSA can raise Change Proposals (CPs) to amend the Agreement with the consent of other Parties and (where applicable) the Authority.

## The DCUSA currently requires Distribution Network Operators (DNOs) to determine an F Factor for each Extra High Voltage (EHV) Distribution Charging Methodology (EDCM) embedded generator based on the criteria set down in Engineering Recommendation P2/6 – ‘Security of Supply’ (ER P2/6) and Engineering Technical Report 130 – ‘Application Guide for Assessing the Capacity of Networks Containing Distribution Generation’ (ETR130). The F Factor is determined based on a site-specific assessment of the contribution to network security of each EDCM embedded generator, taking into account availability and the operating regime, alongside intermittency.

## EDCM embedded generators are deemed to be eligible to receive charge one credits (unit rate credits applicable in the DNO’s peak ‘super-red’ period, calculated based on a power flow analysis of the DNO’s network) if they have a non-zero F Factor, and are deemed not eligible to receive charge one credits if they have a zero F Factor.

## This change seeks to improve the transparency around the determination of the eligibility of EDCM embedded generators to receive charge one credits.

#### Why?

## This CP has been raised following a concern raised by embedded generators that there is a lack of transparency and potential lack of commonality in the method by which DNOs determine the F Factor, and consequently whether prospective sites will be eligible for charge one credits.

## How?

## The Working Group have reviewed the responses to the first consultation and developed a number of solutions. A further refinement to Option 1 and two further Options (1A and 2B) after discounting Option 2. The three  potential solutions are proposed:

- Option 1 - DNOs to set the ‘proportion eligible for charge one credits’ to one for all non-intermittent generators, and according to the status quo for intermittent generators (i.e. set to one if a non-zero F Factor has been assigned and set to zero otherwise). The refinement being that the criteria for mixed sites will be non-intermittent if the installed capacity of the non-intermittent generation is greater than or equal to 50% of the Maximum Export Capacity, otherwise it will be classed as intermittent; or

- Option 1A – Is the same as Option 1 but the methodology used for mixed sites is based on would be based on the non-intermittent generation installed capacity as a percentage of the Maximum Export Capacity being the proportion eligible for Charge 1 credits; or

- Option 2B - clarifying the use of F Factors in the power flow modelling to explicitly require that the DNO should determine whether the generator is sufficiently reliable to provide a contribution to security of supply should the need arise, not whether the generator is currently making a contribution. This would involve assigning the F Factor in line with the process laid out in P2/6 and ETR130 without considering the location of the generator.

- Governance

#### Justification for Part 1 Matter

## This CP is considered a Part 1 Matter as it is likely to have a significant impact on competition in the generation of electricity (DCUSA Clause 9.4.2 (B)).

#### Current Next Steps

## This Consultation will be issued for a period of three weeks.

- Why Change?

#### Background of DCP 313

## This CP seeks to address a Distribution Charging Methodology Forum (DCMF) Methodologies Issue Group (MIG) issue raised in November 2016 which identified potential differences in the application of generation credits to EDCM embedded generators across different DNOs.

  - The concern raised by embedded generators was that there is a lack of transparency and potential lack of commonality in the method by which DNOs determine the F Factor, and consequently whether prospective sites will be eligible for charge one credits.

  - DCP 291 – ‘Application of Generation Credits to EDCM Customers’ was raised to resolve this issue, by making all EDCM embedded generators eligible for charge one credits regardless of the F Factor assigned. The DCP 291 Working Group subsequently agreed that this was not the best available solution to the issues raised, which led to the withdrawal of DCP 291 and the creation of this CP.

  - Originally, there were two proposed solutions for this change, both of which would improve the transparency around the eligibility for charge one credits, with the second solution also achieving a transparent approach for the assignment of F Factors to EDCM embedded generators.

Option 1 – Proportion eligible for credits set according to technology type rather than based on the F Factor assigned for non-intermittent generators

  - Amend legal text Schedules 17 and 18 to require DNOs to set the ‘proportion eligible for charge one credits’ field to one for all non-intermittent generators and according to the status quo for intermittent generators (i.e. set to one if a non-zero F Factor has been assigned and set to zero otherwise). This would lead to:

- All intermittent EDCM embedded generators remaining unchanged;

- Non-intermittent EDCM embedded generators which the DNO has determined do not support the network in line with ETR130 (which have zero F Factor) being eligible for charge one credits where they are currently not; and

- Non-intermittent EDCM embedded generators which the DNO has determined do support the network in line with ETR130 (which have non-zero F Factor) remaining eligible for charge one credits.

  - This will provide greater transparency to non-intermittent EDCM embedded generators of the process by which the DNO will determine eligibility for charge one credits, and so enable them to more easily predict the likely charges/credits they will face when deciding where to site plant.

  - This option will only improve transparency in the eligibility for credits by divorcing the eligibility criteria for non-intermittent EDCM embedded generators from the site-specific assessment carried out to determine the F Factor. The assignment of the F Factor will remain unchanged by this option, and so a non-intermittent EDCM embedded generator may still be assigned a zero F Factor if they are deemed by the DNO to not support the network.

  - As a result, there is a possibility that a non-intermittent EDCM embedded generator which is deemed not to support the network will be awarded credits. This is unlikely, as a generator which does not support the network is likely to be in an area of low demand, and hence charge one is likely to be zero. Nonetheless, it is possible that charge one will be non-zero, and so a generator which does not offset reinforcement costs could be awarded credits under this solution.

  - There is also a risk under this solution that a scenario where a non-intermittent EDCM embedded generator would be awarded higher credits if it were deemed not to support the network by the DNO than if it were deemed to support the network by the DNO. This is because in the case where the generator is deemed to support the network, it will have a non-zero F Factor, and so will be assumed to be generating at the time of peak demand (in line with the load flow methodology), and so the time to reinforcement on the local network will be longer and charge one lower. If the same generator were deemed not to support the network, it would be assigned a zero F Factor, and so assumed to not be generating at the time of peak demand (in line with the load flow methodology), and so the time to reinforcement on the network will be shorter and the charge one higher. Under this option, the generator in both scenarios would be eligible for charge one credits (because it is non-intermittent) but charge one would be higher in the case where the generator is deemed not to support the network, and so the credits the generator received would be higher in this scenario.

Option 2 – F Factor assigned based on technology type with no site-specific assessment

  - Amend legal text in Schedules 17 and 18 to no longer reference P2/6 when assigning the F Factor, but rather to include a modified table 2-1 from P2/6 in the EDCM, and so set the F Factor based only on technology type with no site-specific assessment. This would lead to:

- Intermittent EDCM embedded generators which have zero F Factor remaining unchanged (this would be the majority of intermittent generators);

- Intermittent EDCM embedded generators with a non-zero F Factor being assigned a zero F factor and so becoming ineligible for charge one credits (this would be the minority of intermittent generators);

- Non-intermittent EDCM embedded generators which the DNO has determined do not support the network in line with the ETR130 (which have zero F Factor) being reassigned a non-zero F Factor and so becoming eligible for charge one credits; and

- Non-intermittent EDCM embedded generators which the DNO has determined do support the network in line with ETR130 (which have non-zero F Factor) being assigned the same F Factor and so remaining eligible for charge one credits.

  - This will provide greater transparency to all EDCM embedded generators (compared to option one which only provides greater transparency for non-intermittent EDCM embedded generators) of the process by which the DNO will determine eligibility for charge on credits, and so enable them to more easily predict the likely charges/credits they will face when deciding where to site plant. This option will also improve transparency in the determination of the F Factor.

  - The possibility identified in option one that a generator which is deemed not to support the network will be awarded credits remains a risk under option two. As with option one, this is unlikely, as a generator which does not support the network is likely to be in an area of low demand, and hence charge one is likely to be zero. Nonetheless, it is possible that charge one will be non-zero, and so a generator which does not offset reinforcement costs could be awarded credits under this option.

  - The risk identified under option one of a scenario where a non-intermittent EDCM embedded generator would be awarded higher credits if it were deemed not to support the network by the DNO than if it were deemed to support the network by the DNO does not exist under this option. This is because both generators would be assigned the same F Factor based on table 2-1 from P2/6, and so both would have the same charge one calculated.

  - For both options, the Working Group would need to determine whether the increased transparency and commonality outweigh the potential loss in cost-reflectivity.

  - However, following on from the responses received to the first DCP 313 consultation that was issued to DCUSA Parties on 23 February 2018, it was decided that there should be further development on the solutions of the Change Proposal. This included removing Option 2 and developing two further options (1A and 2B). More information on this can be found in Section 4 below.

- Working Group Assessment

#### DCP 313 Working Group Assessment

## The DCUSA Panel established a Working Group to assess DCP 313. This Working Group consists of DNO and supplier representatives, with an Ofgem observer. Meetings were held in open session and the minutes and papers of each meeting are available on the DCUSA website – www.dcusa.co.uk.

## Consultation One

  - To aid the further development of the solution for this CP, the Working Group issued a consultation to Parties on 23 February 2018. The aim of the first consultation was to ask the industry for their views on the principles of the change and which original solution they preferred. There were eight respondents to the first consultation comprising of five DNOs, two Generators and one Supplier. A copy of the first consultation and the Working Group conclusions can be found as Attachment 4.

  - The majority of respondents agreed that they understood and agreed with the principles and intent of the change. However, one DNO respondent voiced that they did not agree or support the change. The Working Group discussed the DNOs response and decided that the party had interpreted the intent of the change differently than that of the Working Group.

  - Respondents to the consultation also raised that the legal text did not refer to what would happen when there is an intermittent and non-intermittent generator in the same location and how their tariff is determined. The Working Group agreed that this was a valid concern

  - Generators responded to highlight that some DNOs are referring to ETR130 and P2/6 more widely to determine the proportion eligible for generation credits. They stated that some DNOs will say that unless the network relies on Distributed Generation (DG) to meet the standard laid down in P2/6 that the credits should be set to zero by setting the F Factor to zero. This means that even when Table 2-1 states a non-zero F Factor and the charge 1 is non-zero the generation credits are set to zero as the F Factor is overridden and set to zero. For generators, this results in the prevention of cost reflective generation credits being signalled to generators and makes it impossible for potential DG to calculate the potential generation credits.

  - Even though this was the generators opinion, other responses to the consultation highlighted that inclusion of a table that includes specific technology types would not be sufficient as it would not future-proof potential new technologies even though a default value was available within Option 2.

  - On which option did parties support, one party did not support the introduction of discrimination based on technology type into the EDCM. In their view technology type is not a driver of network requirements and therefore does not drive costs. For consistency of application the Party favoured the introduction of a forum where f-factor calculations could be discussed amongst peers, and where best practice could be agreed and shared. This group could be a special meeting of the DCMDG, perhaps on an annual basis with the invite extended to network representatives familiar with the f-factor calculations for their network areas. Whilst there was support in the Working Group for such an initiative it was suggested that this could be progressed outside of this change.

  - Parties that supported Option 1 cited that the change was unambiguous, provided consistency, and simplicity. There was some support for option 2 however one party stated that it is not necessary to assess the underlying characteristics of the controllable generator as the total revenue available to the generator will be operating hours x credit rate. A generation technology which is either less reliable or unable to maintain its maximum output will receive a lower credit by virtue of its lower output over the peak.  However, if contributing at peak its MWh are as good as any other plant’s. The majority of the responses supported Option 1.

Working Group Conclusions and next steps

  - The Working Group identified that there were a number of areas of further work having discussed the Parties’ responses to the first consultation:

- The intent of the change proposal;

- Decide which Option should progress further;

- Refine the solution to cater for mixed sites; and

Consider whether any alternative solutions are required. Intent

  - The Working Group agreed to amend the intent to provide further clarity and seek the DCUSA Panel approval to the amendments made.

  - A paper was submitted to the DCUSA Panel meeting in April 2018. The Panel, whilst recognising the concern raised by one party, where comfortable with the original intent and as such it was sufficient when trying to amend the eligibility criteria for EDCM generation credits. The paper was therefore rejected and the original intent stands.

Option to progress

  - The Working Group by a majority supported the progression of Option 1 in preference to Option 2, however accepted that this option still needed further development to cover mixed sites.

Mixed sites

  - The Working Group discussed the issue of mixed sites. It was noted by the Working Group that this covered off for Low Voltage (LV) and High Voltage (HV) connections within the Licence Condition 14 statement whereby the dominant technology determines whether the site is intermittent or non-intermittent. It was agreed that a similar clause should be added to Option 1 to that should be included to cover Extra High Voltage (EHV) connections and mixed sites, providing greater transparency in this area.

  - An alternative approach (option 1b) was suggested that looked whether the solution should be a non-binary one and that the eligibility for charge 1 credit should be based on the installed capacity of the non-intermittent as a percentage of the Maximum Export Capacity.

  - It was also noted by the Working Group that this may help future proof the change proposal since there are only a handful of sites at the moment and in some instances these are billed separately so currently not an issue. The Working Group agreed to undertake a Request for Information to determine the number of such sites and what process DNOs undertake to determine the F factor.

Request for Information

  - The Working Group asked for information on the following:

- How many sites currently have intermittent and non-intermittent generation on the same site;

- What is the DNO process under the EDCM in determining credits for intermittent and non-intermittent generators at the same site;

- Would the F-factor be reduced on a mixed site resulting in a reduction in the credit provided and if so is there a similar reduction in the credit provided e.g. If the F Factor was reduced by 50% would it reduce the credit by 50% too; and

- On sites where there is intermittent and non-intermittent generation and each technology type is fed by a separate MPAN would the DNOs provide a credit associated with each MPAN or would the credit be associated with the site.

  - The Working Group reviewed the RFI responses received from five of the six DNOs and noted that there are only four mixed sites, three of which are actually charged separately so that impacted by the definition within the legal text for Option 1. RFI responses are in Attachment 5.

  - In response to Question two on how the calculation of the F factor would be made, the majority referred to P2/6 and ETR130 together with a site assessment. One response went further indicating that they would assess the site and use an average of the generic F factors based on the MEC proportionate to both intermittent and non-intermittent generation. It was clarified at the meeting that this was for new connections.

  - There was a mixed response to the question as to whether the F factor would be reduced on a mixed site resulting in a reduction in the credit provided and whether such a reduction would result in a similar reduction in the credits provided. The expectation was that the F factor would be reduced but the credit would be subject to the outcome of the load flow analysis. The Working Group agreed that the reduction is not likely to be linear.

  - On the final question as to whether mixed generation sites which have a separate MPANs would result in separate treatment, the response again was mixed. Some do treat them as separate and as such charge each one separate, some treat them as one site and one DNO follows the Elexon guidelines. The Working Group noted that it may be an expensive solution to charge each one separately due to the impact on DNO systems to accommodate this and the corresponding impact on demand connections.

  - The Working Group agreed to progress with two solutions for mixed sites as indicated in paragraph 4.13 and 4.14 above and seek industry views.

Alternative options

  - The Working Group sought additional information from a respondent who felt that DNOs are non-compliant with the DCUSA and the Charging Methodologies, as suggested in their response to question three of the DCP 313 consultation.

  - The response was:

“Some DNO’s are referring to ETR130 and P2/6 more widely to determine whether to zero the proportion eligible for generation credits. Engineering recommendation P2/6 is a guidance document on system planning and network capacity requirements and details the minimum standard for the security of supply of a network. Where a network does not meet the requirements of P2/6 without the contribution of DG the DNO may use the value in Table 2-4 to determine how much of the DG’s capacity can be taken into account in assessing the adequacy of the network.

Certain DNO’s are using this to say that unless the network relies on DG to meet the standard laid down in P2/6 that the generation credits should be set to zero by setting the F factor to zero. This means that even when Table 2-4 states a non-zero F factor and the Charge 1 is non-zero (indicating future demand led reinforcement) the generation credits are set to zero as the F facto is overridden and set to zero.

It is our view that this is an incorrect application of the requirements of Schedule 17 of DCUSA and results in the prevention of cost reflective generation credits being signalled to generators. It also makes it impossible for potential DG to calculate the potential generation credits as they rely, in certain DNO regions, on a subjective assessment and application of the F factor.”

  - The Working Group discussed that, from the interpretation of the response, it seems as though the insertion of Table 2-4 should be copied from P2/6 into DCUSA which would align with Option two of the proposed solutions.

  - It was suggested that an alternative third option should be developed to strengthen the legal text and align with the generator interpretation without including Table 2-4 from P2/6 which had caused concerns in the consultation responses.

  - There was a concern that the transition from P2/6 to P2/7 (if approved by the Authority) would remove the differences between intermittent and non-intermittent generation which would mean that further work would need to be done in this area.

  - The Working Group agreed that the development of an “Option 2B” would warrant a further consultation to be issued to Parties as the legal text would be completely different to the first consultation. One Working Group member highlighted that the majority of the respondents to the first consultation were supportive of Option One and the new option could change industry views and therefore a consultation should be issued. The Working Group to undertake a second consultation covering the inclusion of the criteria for mixed sites (resulting in an updated Option 1 and a new Option 1A) together with an alternative option 2B.

## Second Consultation

  - Following on from the responses of the first consultation and the analysis undertaken, the Working Group agreed that they would need to further consult again to obtain industry feedback on the newly developed Options 1, 1A and 2B and distinguish whether there would be any detrimental impacts if any of the approached were implemented. Details of each solution can be found below.

## Option 1

  - This solution will require DNOs to populate the ‘proportion eligible for charge one credits’ field based on technology type rather than on F Factor, i.e. set to one for all non-intermittent EDCM embedded generators and according to the status quo for all intermittent EDCM embedded generators (i.e. set to one if a non-zero F Factor has been assigned and set to zero otherwise).

  - The implementation of Option 1 would lead to:

- All intermittent EDCM embedded generators remaining unchanged;

- Non-intermittent EDCM embedded generators which the DNO has determined do not support the network in line with ETR130 (which have zero F Factor) being eligible for charge one credits where they currently are not;

- Non-intermittent EDCM embedded generators which the DNO has determined do support the network, in line with ETR130 (which have non-zero F Factor) remaining eligible for charge one credits; and

- Generation will be determined to be non-intermittent if the installed capacity of the non-intermittent generation is greater than or equal to 50% of the Maximum Export Capacity.

## Option 1A

  - Is the same as Option 1 but the methodology used for mixed sites is based on would be based on the non-intermittent generation installed capacity as a percentage of the Maximum Export Capacity being the proportion eligible for Charge 1 credits.

  - This would result in the process for determining generation credits being non-binary and should provide the industry with a more future-proofed solution.

## Option 2B

  - Option 2B would involve clarifying the use of F Factors in the power flow modelling to explicitly require that the DNO should determine whether the generator is sufficiently reliable to provide a contribution to security of supply should the need arise, not whether the generator is currently making a contribution. This would involve assigning the F Factor in line with the process laid out in P2/6 and ETR130 without considering the location of the generator.

  - This solution aims to strengthen the legal text and align with the generators interpretation of the intent of the change (as described in section 4.5 above) without including table 2-1 from P2/6.

  - To ensure that this option is fully developed, the Working Group have instructed TNEI to conduct a qualitative assessment on the power flow model of the EDCM to determine the impact that this change would have on the current generator credits. Results of this impact assessment can be found in paragraph 7.10 below.

## Q1: Whilst considering the ongoing work being conducted by Ofgem on Access and Forward-Looking Charges, which solution option do you support and why?

- Legal Text

## DCP 313 Proposed Legal Text

  - The proposed changes to Schedules 17 and 18 for Option 1, Option 1A and Option 2B can all be found within Attachment 2.

Option 1

  - Option 1 involves a simple change to the legal text, to make all non-intermittent EDCM embedded generators eligible for charge one credits regardless of the F Factor assigned. In regards to mixed generation sites, this option would determine all generation associated with that Connectee non-intermittent if the installed capacity of the non-intermittent generation is greater than or equal to 50% of the Maximum Export Capacity, otherwise it will be determined to be intermittent.

Option 1A

  - Option 1A also involves a simple change to the legal text to make all non-intermittent EDCM embedded generators eligible for charge on credits, regardless of the F Factor assigned. However, for cases where there are mixed generation sites (and the DNO bills them together) the proportion eligible for charge 1 credits is equal to the non-intermittent generation installed capacity as a percentage of the Maximum Export Capacity.

Option 2B

  - Option 2B changes the legal text so that the F Factor would need to be assigned on a site-specific basis and would be determined by using P2/6 and ETR 130 to conduct the site-specific assessment.

  - The DNO Party would need to determine the percentage of the generator’s declared net capacity which would be considered to support security of supply in the event that the network alone did not.

  - The eligibility of charge 1 credits would be decided on whether the P2/6 site assessment determined whether the embedded generation sufficiently supported the network or not.

  - Mixed sites would be catered by the site-specific assessment so no need to include any addition criteria.

Q2: Do you have any comments on all options of the proposed legal text?

- Relevant Objectives

## Assessment Against the DCUSA Objectives

  - For a DCUSA CP to be approved it must be demonstrated that it better meets the DCUSA Charging Objectives.

  - The Proposer believes that both proposed solutions will better facilitate Charging Objective two by providing greater transparency to EDCM embedded generators of the process by which the DNO will determine eligibility for charge one credits, and so enable them to more easily predict the likely charges/credits they will face when deciding where to site plant.

  - Option 2B will facilitate objective two more strongly by improving transparency in the assignment of the F Factor alongside the eligibility for credits; whilst Option 1 will only improve transparency in the eligibility for credits for non-intermittent EDCM embedded generators by divorcing eligibility from the site-specific assessment carried out to determine the F Factor.

  - However, both proposed solutions will have a small detrimental impact against charging objective three, as there is the possibility that EDCM embedded generators which are deemed not to support the network by the DNO could be awarded credits.

  - Option 1 would have a slightly worse detrimental impact than Option 2B, due to the issue where an EDCM embedded generator which is deemed to support the network could receive lower credits than an equivalent EDCM embedded generator which is deemed to not support the network.

| DCUSA Charging Objectives | Identified impact |
| --- | --- |
| 1 that compliance by each DNO Party with the Charging Methodologies facilitates the discharge by the DNO Party of the obligations imposed on it under the Act and by its Distribution Licence | None |
| 2 that compliance by each DNO Party with the Charging Methodologies facilitates competition in the generation and supply of electricity and will not restrict, distort, or prevent competition in the transmission or distribution of electricity or in participation in the operation of an Interconnector (as defined in the Distribution Licences). | Positive |
| 3 that compliance by each DNO Party with the Charging Methodologies results in charges which, so far as is reasonably practicable after taking account of implementation costs, reflect the costs incurred, or reasonably expected to be incurred, by the DNO Party in its Distribution Business. | Negative |
| 4 that, so far as is consistent with Clauses 3.2.1 to 3.2.3, the Charging Methodologies, so far as is reasonably practicable, properly take account of developments in each DNO Party’s Distribution Business | None |
| 5 that compliance by each DNO Party with the Charging Methodologies facilitates compliance with the Regulation on Cross-Border Exchange in Electricity and any relevant legally binding decisions of the European Commission and/or the Agency for the Co-operation of Energy Regulators. | None |
| 6 that compliance with the Charging Methodologies promotes efficiency in its own implementation and administration. | None |

Q3: Which of the DCUSA Charging Objective does this CP better facilitate? Please provide supporting comments.

- Impacts & Other Considerations

#### Does this Change Proposal impact a Significant Code Review (SCR) or other significant industry change projects, if so, how?

## This CP does not have any impacts on the Target Charging Review (TCR) SCR. However, the Working Group is aware of DCP287 - 'Generation credits in the EDCM’ that may impact this change proposal. It is understood that DCP287 is still developing their solution and conducting modelling updates and therefore, DCP 313 will still progress.

  - The Working Group are aware of the recent Distribution Code (Dcode) consultation for DCRP/18/03/PC that is looking to replace P2/6 with a proposed P2/7.

  - The proposed implementation date of the proposed P2/7 is quarter 2 of 2018 dependent on Authority approval and hence the Working Group decided to continue with DCP 313 in its current form. The main impact being that references to the definitions of intermittent and non-intermittent generations together with carious tables are being removed from P2/6. It was noted that they are present within ETR130. Should P2/6 be replaced with P2/7, there will be a need for a further CP to be raised as P2/6 is referenced in other parts of the DCUSA not relating to this CP. The Working Group felt that a generic review of DCUSA would be more appropriate should P2/7 be approved rather than await f the outcome on P2/7 and as such put this change on hold.

#### Consumer Impact

#### Option One

## The Working Group conducted a request for information (RFI) from DNOs to determine the impacts on EDCM tariffs for both options. The outcome from that RFI is in Attachment 4.

  - The majority of DNO areas provided impacts on their EDCM tariffs for option one and the impact assessment showed that for the majority of DNO areas there would be no impact on EDCM tariffs if option one was accepted. However, one DNO area highlighted that there would be an impact on customers in their area if option one was accepted.

  - It was noted that in this area, 33 generators would be affected by the changes of option one, which would mean a difference of between 0.501p/kWh and 10.452p/kWh on the super-red unit rate export tariffs for the generators affected.

#### Option 1A

  - The impacts of for Option 1A would be the same as option 1 since the only difference between the two was the approach taken to mixed sites and the RFI indicated that were such sites have been identified they are being billed separately.

#### Option 2B

  - As the solutions were developed, the Working Group identified a need to conduct a qualitative impact assessment on the powerflow model contained in the EDCM to be able to understand the impacts on their customers if Option 2B was accepted.

  - To conduct this the DCUSA Panel agreed to release money from the DCUSA budget to be able to instruct TNEI to conduct the analysis and provide a report to the Working Group.

  - Results to be included here.

#### Environmental Impacts

## In accordance with DCUSA Clause 11.14.6, the proposer assessed whether there would be a material impact on greenhouse gas emissions if DCP 313 were implemented. The proposer did not identify any material impact on greenhouse gas emissions from the implementation of this CP.

#### Engagement with the Authority

## Ofgem has been fully engaged throughout the development of DCP 313 as an observer on the Working Group.

Q4: Are you aware of any wider industry developments that may impact upon or be impacted by this CP?

- Implementation

## The proposed implementation date for DCP 313 is 01 April 2021. Respondents are invited to consider whether they agree with the proposed implementation date.

Q5: The proposed implementation date for DCP 313 is 01 April 2021. Do you agree with the proposed implementation date?

- Consultation Questions

## The Working Group is seeking industry views on the following consultation questions:

| Number | Questions |
| --- | --- |
|  | Whilst considering the ongoing work being conducted by Ofgem on Access and Forward-Looking Charges, which solution option do you support and why? |
|  | Do you have any comments on all options of the proposed legal text? |
|  | Which of the DCUSA Charging Objective does this CP better facilitate? Please provide supporting comments. |
|  | Are you aware of any wider industry developments that may impact upon or be impacted by this CP? |
|  | The proposed implementation date for DCP 313 is 01 April 2021. Do you agree with the proposed implementation date? |

## Responses should be submitted using Attachment 1 to dcusa@electralink.co.uk no later than, XX XXX XX.

## Responses, or any part thereof, can be provided in confidence. Parties are asked to clearly indicate any parts of a response that are to be treated confidentially.

#### Attachments

Attachment 1 – DCP 313 Consultation Response Form

Attachment 2 – DCP 313 Proposed Legal Text

Attachment 3 – DCP 313 Change Proposal

Attachment 4 – Consultation 1 responses

Attachment 5 – RFI responses

Attachment 6 – Impact Assessments

http://www.dcode.org.uk/consultations/open-consultations/