| DCUSA Consultation |  | At what stage is this document in the process? |
| --- | --- | --- |
| DCP 305: LDNO Boundary Level Definitions in the EDCM Date Raised: 12th July 2017 Proposer Name: Andrew Enzor Company Name: Northern Powergrid Company Category: DNO |  | 01 – Change Proposal. 02 – Consultation. 03 – Change Report. 04 – Change Declaration. |
| The intent of this change proposal is to amend the definitions and application of DNO/LDNO boundaries under the EDCM to avoid instances of double- and/or non-charging for certain assets. |  |  |
|  | The Workgroup recommends that this Change Proposal should: - proceed to Consultation ## Parties are invited to consider the questions set in section 10 and submit comments using the form attached as Attachment 1 to dcusa@electralink.co.uk by xx xxxx 2017. DCP 305 has been designated as a Part 1 Matter and a standard change. The Working Group will consider the consultation responses and determine the appropriate next steps for the progression of the Change Proposal (CP). |  |
|  | Impacted Parties: Predominantly DNOs and LDNOs; minor impact on suppliers from revenue matching to correct any DNO revenue surplus/shortfall generated. |  |
|  | Impacted Clauses: Schedules 17 and 18, sections 24, 25 and 26 |  |

Guidance On The Use Of This Template:

Code Administrators will produce this Report using the original proposal as the source.

The Workgroup will verify all of the information provided, adding the Impact Assessment.

| Contents 1. Summary 3 2 Governance 3 3 Why Change? 4 4 Working Group Assessment 4 5 Legal Text and Spreadsheet Template 5 6 Relevant Objectives 7 7 Impacts & Other Considerations 7 8 Implementation 9 9 Consultation Questions 9 Timetable The timetable for the progression of the CP is as follows: #### Change Proposal timetable Change Proposal timetable:. #### Activity; #### Date. Initial Assessment Report Approved by Panel; 19 July 2017. Consultation issued to Parties; TBC. Change Report issued to Panel; TBC. Change Report issued for Voting; TBC. Party Voting Ends; TBC. Change Declaration Issued to Parties; TBC. Authority Decision; TBC. Implementation. | Any questions? |
| --- | --- |
|  | Contact: Code Administrator |
|  | DCUSA@electralink .co.uk |
|  | 02074323000 |
|  | Proposer: Andrew Enzor |
|  | andrew.enzor@northernpowergrid.com |
|  | 07834 618994 |

- Summary

#### What?

## The Distribution Connection and Use of System Agreement (DCUSA) is a multi-party contract between electricity Distributors and electricity Suppliers and large Generators. Parties to the DCUSA can raise Change Proposals (CPs) to amend the Agreement with the consent of other Parties and (where applicable) the Authority.

## This change proposal seeks to resolve a defect identified in the Extra-high-voltage (EHV) Distribution Charging Methodology (EDCM) whereby designated EHV properties connected to Licenced Distribution Network Operator (LDNO) networks (i.e. networks operated by either an Independent Distribution Network Operator (IDNO) or a Distribution Network Operator (DNO) operating out of area) will have the potential to be double charged for the use of certain DNO network assets; and designated high-voltage (HV) or low-voltage (LV) properties will have the potential to not be charged at all for the use of certain DNO network assets.

#### Why?

## The definition of the DNO to LDNO boundary under the EDCM has the following defects:

## In respect of embedded LDNO networks with several connectees, including at least one connectee which is a designated EHV property, the current definition leads to the DNO calculating a boundary equivalent tariff for the designated EHV property having applied the customer category at the DNO/LDNO boundary. This results in double-charging for any DNO assets that are for the sole use of the LDNO network, since the LDNO would pay fixed charges in respect of these assets, and then through the boundary category pay again for these DNO network levels as if they were shared assets.

## However, for an LDNO network with a single connectee which is a designated EHV property, the current definition leads to the DNO calculating a boundary equivalent tariff for the designated EHV property having applied the customer category for the LDNO network (i.e. the point on the DNO network where the power flow associated with the LDNO network interacts with the power flows associated with other DNO connectees). This results in the DNO applying the customer category at the point where assets for the sole use of the LDNO network meet the remainder of the DNO network, and so only charging fixed charges in respect of assets for the sole use of the LDNO network.

## In respect of embedded LDNO networks with a single customer which is a designated HV or designated LV property, the current definition leads to the DNO applying the boundary at the point of common coupling for the LDNO network. This results in the DNO charges recovering nothing in respect of assets that are for the sole use of the LDNO network.

## More detail, including a worked example of each issue, is included in section 3 – ‘Why Change’.

#### How?

## The proposed solution is to introduce separate definitions for the LDNO boundary level for the purposes of section 25 and section 26 of the EDCM. Both definitions would operate in the same way irrespective of the number of customers on the LDNO network.

## For section 25 (which relates to LDNO tariffs for designated LV and designated HV properties where the LDNO network would itself qualify as a designated EHV property) the boundary level would be defined by reference to the DNO/LDNO asset boundary.  The section 25 LDNO boundary classification would take the same five values as now (0000, 132kV, 132kV/EHV, EHV and HV plus).  This would ensure that all DNO network levels used are charged for, irrespective of whether they would be classified as sole use under a power flow analysis or not.

## For section 26 (which relates to LDNO tariffs for designated EHV properties) the customer category would be determined by reference to the same criteria as for DNO end users, using the LDNO network point of common coupling defined as the point on the DNO network where the power flow associated with generation and load on that LDNO network may interact with the power flows associated with other customers. The section 26 LDNO boundary classification would take the same 15 four-digit code values as now. This would ensure consistency with the sole use asset definition at paragraph 26.7, so that DNO assets would be charged for once and only once, either as sole use assets or shared assets through the customer category.

Q1: Do you understand the intent of DCP 305?

- Governance

#### Justification for Part 1 Matter

## DCP 305 has been designated as a Part 1 Matter as the proposed change impacts the governance or the change control arrangements under DCUSA.

#### Requested Next Steps

## Following a review of the Consultation responses, the Working Group will work to agree the detail of the solution for DCP 305.

- Why Change?

#### Background of DCP 305

## The definition of the DNO to LDNO boundary under the EDCM has the following defects:

## In respect of embedded LDNO networks with several connectees, including at least one connectee which is a designated EHV property, the current definition (using clauses 24.8 and 26.3) leads to the DNO calculating a boundary equivalent tariff for the designated EHV property having applied the customer category at the DNO/LDNO boundary.

## Clause 24.8: “IDNO Party Distribution Systems are split into 15 categories based on the network level of the boundary between the host DNO Party and the IDNO Party, and whether or not higher network levels are used by the IDNO Party.”

## Clause 26.3: “For the purposes of calculating the boundary-equivalent portfolio EDCM tariffs, each EDCM Connectee on the IDNO Party’s network would be assigned the demand Connectee category relating to the 15 IDNO Party boundary categories.”

## This may result in double-charging for any DNO assets that are for the sole use of the LDNO network, since the LDNO would pay fixed charges in respect of these assets, and then through the boundary category pay again for these DNO network levels as if they were shared assets.

## However, for an LDNO network with a single connectee which is a designated EHV property, the current definition (using clause 24.6) leads to the DNO calculating a boundary equivalent tariff for the designated EHV property having applied the customer category for the LDNO network (i.e. the point on the DNO network where the power flow associated with the LDNO network interacts with the power flows associated with other DNO connectees).

## Clause 24.6: “Where the IDNO Party’s Distribution System only has one Connectee (whether a designated EHV property or not), the network level of the boundary between the host DNO Party and IDNO Party is determined by reference to the Point of Common Coupling. The Point of Common Coupling is determined in the same way as it is for an EDCM Connectee connected directly to the host DNO Party’s network.”

## This results in the DNO applying the customer category at the point where assets for the sole use of the LDNO network meet the remainder of the DNO network, and so only charging fixed charges in respect of assets for the sole use of the LDNO network.

## For example, consider an LDNO taking a supply from a dedicated DNO 132kV cable from a GSP (i.e. where the DNO 132kV cable is for the sole use of the LDNO network) and supplying one customer which is a designated EHV property through its embedded system, as shown in Figure 1.

Figure 1 – LDNO Network with one Designated EHV Property only

## When calculating a boundary equivalent tariff, the DNO would assign the customer category ‘by reference to the Point of Common Coupling’ (clause 24.6) marked as point ‘A’ in Figure 1, i.e. 0000 – ‘Point of Common Coupling at the GSP, whether the GSP is shared or not’, and so the customer would be charged for the DNO 132kV cable from point ‘A’ to point ‘B’ as a sole use asset only.

## Now consider another customer which is a designated EHV property (or indeed any other connectee) connecting to that LDNO network, as shown in Figure 2.

Figure 2 – LDNO Network with two Designated EHV Properties

## When calculating boundary equivalent tariffs the DNO would assign both customers category ’the demand Connectee category relating to the 15 IDNO Party boundary categories’ (clause 26.3) which are ’based on the network level of the boundary between the host DNO Party and the IDNO Party’ (clause 24.8) marked as point ‘B’ in Figure 2, i.e. 1000 – ‘In England or Wales only, Point of Common Coupling at a voltage of 132 kV, unless the Connectee qualifies for category 0000’, and so the customer would be charged for the DNO 132kV cable from point ‘A’ to point ‘B’ as both a sole use asset and a shared asset.

## In respect of embedded LDNO systems with a single customer which is a designated HV or designated LV property, the current definition (using clause 24.6) leads to the DNO applying the boundary at the point of common coupling for the LDNO network. This results in the DNO charges recovering nothing in respect of assets that are for the sole use of the LDNO network.

## For example, consider an LDNO taking a supply from a dedicated DNO 132kV/11kV substation fed directly from a GSP and supplying a single HV network customer, as shown in Figure 3.

Figure 3 – LDNO Network with one Designated HV Property only

## When determining which tariff to apply, the DNO would assign the customer category ’by reference to the Point of Common Coupling‘ (clause 24.6) marked as point ‘A’ in Figure 3, i.e. 0000 – ‘Point of Common Coupling at the GSP, whether the GSP is shared or not’, so this customer would be assigned the ‘LDNO 0000: HV HH Metered’ tariff. This results in the customer not being charged at all for use of assets between points ‘A’ and ‘B’ (the DNO’s 132kV circuits and the DNO’s 132kV/11kV substation).

## Now consider another HV network customer (or indeed any other connectee) connecting to the LDNO network, as shown in Figure 4.

Figure 4 – LDNO Network with two Designated HV Properties

## Clause 24.6 would no longer apply and so the DNO would assign both customers the category based on the DNO/LDNO boundary, marked as point ‘B’ Figure 4, i.e. 1001 – ‘Point of Common Coupling at a voltage of less than 22 kV on the secondary side of a substation whose primary side is attached to a 132kV distribution circuit’. So both customers would be assigned the ‘LDNO HVplus: HV HH Metered’ tariff and would be charged for the assets between points ‘A’ and ‘B’.

Q2: Do you agree with the proposer that the interpretation that this is common practice, or that following the DCUSA as it currently stands results in double charging?

Q3: Do you have any other such customers that fall into any of these categories?

Q4: Are there any such instances where a customer is connected to the IDNO network where you are charging for sole use assets and a shared asset?

Q5: What will the impacts be for each of the scenarios? Please provide rationale.

Q6: Are there many instances of these scenarios occurring, and should a derogation be sought if the impacts are material? Please provide rationale.

- Code Specific Matters

#### Reference Documents

  - n/a

- Working Group Assessment

#### DCP 305 Working Group Assessment

## The DCUSA Panel established a Working Group to assess DCP 305. This Working Group consists of DNO representatives and an Ofgem observer. Meetings were held in open session and the minutes and papers of each meeting are available on the DCUSA website – www.dcusa.co.uk.

  - <To be completed by the Working Group>

- Legal Text

## The DCP 305 Legal Text can be found as Attachment X.

Q7: Do you have any comments on the proposed legal text?

- Relevant Objectives

## Assessment Against the DCUSA Objectives

  - For a DCUSA Change Proposal to be approved it must be demonstrated that it better meets the DCUSA Objectives. The Working Group is interested in parties’ views on which of the following DCUSA General Objectives are better facilitated by this change and why.

Q8: Which DCUSA Charging Objectives does the CP better facilitate? Please provide supporting comments.

DCUSA General Objectives:

Charging Objective 1 - that compliance by each DNO Party with the Charging Methodologies facilitates the discharge by the DNO Party of the obligations imposed on it under the Act and by its Distribution Licence

Charging Objective 2 - that compliance by each DNO Party with the Charging Methodologies facilitates competition in the generation and supply of electricity and will not restrict, distort, or prevent competition in the transmission or distribution of electricity or in participation in the operation of an Interconnector (as defined in the Distribution Licences)

Charging Objective 3 - that compliance by each DNO Party with the Charging Methodologies results in charges which, so far as is reasonably practicable after taking account of implementation costs, reflect the costs incurred, or reasonably expected to be incurred, by the DNO Party in its Distribution Business

Charging Objective 4 - that, so far as is consistent with Clauses 3.2.1 to 3.2.3, the Charging Methodologies, so far as is reasonably practicable, properly take account of developments in each DNO Party’s Distribution Business

Charging Objective 5 - that compliance by each DNO Party with the Charging Methodologies facilitates compliance with the Regulation on Cross-Border Exchange in Electricity and any relevant legally binding decisions of the European Commission and/or the Agency for the Co-operation of Energy Regulators.

Charging Objective 6 - that compliance with the Charging Methodologies promotes efficiency in its own implementation and administration

- Impacts & Other Considerations

#### Does this Change Proposal impact a Significant Code Review (SCR) or other significant industry change projects, if so, how?

## No

#### Consumer Impacts

## N/A

#### Environmental Impacts

## In accordance with DCUSA Clause 11.14.6, the Working Group assessed whether there would be a material impact on greenhouse gas emissions if DCP 305 were implemented. The Working Group did not identify any material impact on greenhouse gas emissions from the implementation of this CP.

#### Engagement with the Authority

## Ofgem has been fully engaged throughout the development of DCP 305 as an observer on the Working Group.

Q9: Are you aware of any wider industry developments that may impact upon or be impacted by this CP?

Q10: Are there any alternative solutions or unintended consequences that should be considered by the Working Group?

- Implementation

## It is proposed that the body text changes of DCP 305 are implemented on 01 February 2018.

Q11: The proposed implementation date for DCP 305 is the 01 February 2018. Do you agree with the proposed implementation date?

- Consultation Questions

## The Working Group is seeking industry views on the following consultation questions:

| Number | Questions |
| --- | --- |
|  | ## Do you understand the intent of DCP 305? |
|  | ## Do you agree with the proposer that the interpretation that this is common practice, or that following the DCUSA as it currently stands results in double charging? |
|  | ## Do you have any other such customers that fall into any of these categories? |
|  | ## Are there any such instances where a customer is connected to the IDNO network ## where you are charging for sole use assets and a shared asset? |
|  | ## What will the impacts be for each of the scenarios? Please provide rationale. |
|  | ## Are there many instances of these scenarios occurring, and should a derogation be sought if the impacts are material? Please provide rationale. |
|  | Do you have any comments on the proposed legal text? |
|  | Which DCUSA Charging Objectives does the CP better facilitate? Please provide supporting comments. |
|  | Are you aware of any wider industry developments that may impact upon or be impacted by this CP? |
|  | ## Are there any alternative solutions or unintended consequences that should be considered by the Working Group? |
|  | ## The proposed implementation date for DCP 305 is the 01 February 2018. Do you agree with the proposed implementation date? |

## Responses should be submitted using Attachment 1 to dcusa@electralink.co.uk no later than xx xxxx 2017.

## Responses, or any part thereof, can be provided in confidence. Parties are asked to clearly indicate any parts of a response that are to be treated confidentially.

#### Attachments

Attachment 1 –

Attachment 2 –