| DCUSA Consultation |  | At what stage is this document in the process? |
| --- | --- | --- |
| DCP 287 Generation Credits in the EDCM Raised on the 7 December 2016 as a Standard Change |  | 01 – Change Proposal. 02 – Consultation. 03 – Change Report. 04 – Change Declaration. |
| Purpose of Change Proposal: DCP 287 seeks to amend the calculation of credits for embedded generation in the EDCM to take account of potential cost savings for DNOs that can be attributed to embedded generation in the areas of transmission exit charges, direct costs, indirect costs and network rates. |  |  |
|  | The Workgroup recommends that this Change Proposal should proceed to Consultation ## Parties are invited to consider the questions set in section 10 and submit comments using the form attached as Attachment 1 to dcusa@electralink.co.uk by xx xxxx 2017. The Working Group will consider the consultation responses and determine the appropriate next steps for the progression of the Change Proposal (CP). |  |
|  | Impacted Parties: Distribution Network Operators (DNOs), Distributed Generators and Suppliers |  |
|  | Impacted Clauses: Schedule 17 and 18 (EDCM) |  |

Guidance On The Use Of This Template:

Code Administrators will produce this Report using the original proposal as the source.

The Workgroup will verify all of the information provided, adding the Impact Assessment.

| Contents 1. Summary 3 2 Governance 4 3 Why Change? 4 4 Code Specific Matters 5 5 Working Group Assessment 5 6 Solution and Legal Text 7 7 Relevant Objectives 8 8 Impacts & Other Considerations 9 9 Implementation 9 10 Consultation Questions 9 Timetable The timetable for the progression of the CP is as follows: #### Change Proposal timetable Change Proposal timetable:. #### Activity; #### Date. Initial Assessment Report Approved by Panel; 21 December 2016. Consultation issued to Parties. Change Report issued to Panel; 19 April 2017. Change Report issued for Voting; 21 April 2017. Party Voting Ends; 12 May 2017. Change Declaration Issued to Parties/Authority; 16 May 2017. Authority Decision; 20 June 2017. Implementation; 01 April 2019. | Any questions? |
| --- | --- |
|  | Contact: Dan Fittock |
|  | dan.fittock@electralink.co.uk |
|  | 0792 129 6613 |
|  | Proposer: Johannes Nowak |
|  | johannes.nowa k@mvv.de |
|  | +49 (0)621- 290-4659 |

- Summary

#### What?

## The Distribution Connection and Use of System Agreement (DCUSA) is a multi-party contract between electricity Distributors and electricity Suppliers and large Generators. Parties to the DCUSA can raise Change Proposals (CPs) to amend the Agreement with the consent of other Parties and (where applicable) the Authority.

  - This Change Proposal addresses the issue of whether the calculation of credits for embedded generation in the EDCM should include credits for the avoidance of costs on behalf of the DNO relating to transmission exit charges, direct costs, indirect costs and network rates.

#### Why?

  - The level of credits for embedded generation within the EDCM is determined from the charge 1 that results from a powerflow analysis of the DNOs network. Although this captures future reinforcement costs, it does not necessarily reflect the full costs savings that can be attributed to embedded generation. More cost reflective credits for generators will place incentives on embedded generation that reflect the benefits they bring to network operators.

#### How?

  - The proposed solution is to apply credits to eligible EDCM embedded generators in the areas of:

- Transmission exit charges

- Direct costs

- Indirect costs

- Network rates

- Governance

#### Justification for Part 1 Matter

  - DCP 287 is considered a Part 1 Matter as it affects the level of charges for embedded generation and therefore impacts on competition for embedded generation as specified under 9.4.2 (A). Therefore, DCP 287 will go to the Authority for determination after the voting process has completed.

#### Requested Next Steps

  - Following a review of the Consultation responses, the Working Group will work to agree the detail of the solution for DCP 287.

- Why Change?

#### Background of DCP 287

Transmission Exit Charges

  - The review of embedded benefits undertaken by National Grid in 2013 (charging methodology paper GB ECM-23 Transmission Arrangements for Embedded Generation) determined a value for the avoided cost of embedded generation on the transmission network and excluded the cost of the supergrid transformer as it tends to be fully contributed (ie paid for by the DNO) and is recovered from DNOs via transmission exit charges.

  - Under the EDCM, an award for Transmission exit credits is only paid to qualifying generators that have an agreement with the DNO, the terms of which require the generator, for the purposes of P2/6 compliance, to export power during supergrid transformer (SGT) outage conditions. As most EHV generators do not have this agreement, very few generators receive a credit in this respect.

  - Transmission exit charges recover the capital cost of GSPs on behalf of transmission companies from DNOs. Embedded generation offsets demand at the GSP level and therefore reduces the need for future reinforcement at the GSP. It also increase the amount of spare capacity at the GSP which enables more demand to connect without triggering reinforcement. This principle is accepted within the CDCM where generation receive a credit for offsetting transmission exit charges but not within the EDCM. In addition, the costs of future reinforcement of GSPs is not included in the locational element of the charge (charge 1) as it classified as a zero cost branch due to the ownership lying with the transmission company.

- Direct and Indirect Costs

  - Within the CDCM, embedded generators receive a credit for reducing direct operating costs at voltage levels above the level of connection. This is because they reduce the demand and therefore the level of infrastructure required at higher voltage levels. This results in less reinforcement and also a saving in direct and to a certain extent indirect costs.

  - The level of indirect costs is relatively stable, so the CDCM makes an assumption about the degree to which indirect costs contribute to demand costs or generation credits on a forward looking basis via an operating intensity ratio which is set at 60% within the methodology. Within the EDCM an operating intensity 68% is applied to both direct and indirect costs for the derivation of demand charges.

  - Annex 1 of schedule 17, s8.3 (d) and schedule 18, s7.4 (d) set out the costs to be included when deriving the future reinforcement costs under LRIC and FCP approaches as follows

(d) The typical unit costs used to derive the cost of reinforcement for a Branch shall:

(i) reflect the modern equivalent asset value of reinforcing the particular asset;

(ii) include overheads directly related to the construction activity;

(iii) include building and civil engineering works, in unmade ground.

  - The costs outlined above therefore do not reflect the savings that result from lower direct costs that are realised by the DNO due to the reduction in size of the infrastructure that needs to be maintained by the DNO. In addition, any closely related indirects that may vary with the amount of infrastructure in situ will also decrease. This is reflected in the CDCM, but not in the EDCM.

Network Rates

  - Network rates follow a similar principle. Where less assets are required by the DNO the amount of network rates expenditure by the DNO is reduced. As embedded generation is contributing to the reduced level of costs, it should be rewarded appropriately.

- Code Specific Matters

Reference Documents

n/a.

- Working Group Assessment

#### DCP 287 Working Group Assessment

## The DCUSA Panel established a Working Group to assess DCP 287. This Working Group consists of DNO, Supplier and Ofgem representatives. Meetings were held in open session and the minutes and papers of each meeting are available on the DCUSA website – www.dcusa.co.uk.

  - The Working Group responded to a question why credits are not currently being awarded within the EDCM. Members considered that the CDCM was based on the negative of demand which meant that the credits were included. The EDCM was based on an incremental approach of powerflow analysis being used and by default was not included. The Working Group discussed these points however noted that members couldn’t remember the reasons as to why credits were not awarded to Embedded Generators within the EDCM at the time of the EDCMs inception. The Working Group agreed that it would be beneficial to investigate the discrepancies/differences between the CDCM and EDCM models.

  - The Working Group considered a response to the question of what would be encapsulated within the Charge 1 figure if it was within the CDCM. Members of the Working Group agreed that there is no direct correlation in CDCM but did note that the 500MW model is the closest mechanism which factors the cost of investing in the network on a unit basis.

  - It was questioned if the intent of the proposal is to award credits to both non-intermittent and intermittent generators and if only the categories listed in the CP form will be used for assessing eligibility for credits or if subsequent categories will also be considered. The Proposer noted that they are applied to a subset of plant that supports system security and that if a DNO is not reliant on generator for reinforcement then they won’t be awarded a credit.

  - The Working Group considered a suggestion that over recent years there has been an increase in Embedded Generation and as a result a reduction in direct, indirect and network unit rates should have been seen. On the network rates, they have not seen a correlation between lower demand levels to lower network rates. The Working Group discussed if Embedded Generation was to exactly match demand would indirect costs go down by 60% or would it be closer to 40%. It was agreed to undertake analysis on the increase in Embedded Generation on the network and its impact on direct, indirect and network costs.

  - The Working Group considered a suggestion that in cases where Charge 1 is low then Embedded Generation is of minimal benefit and that in some cases the charge is inflated significantly. The Working Group agreed to consider the principles for each of the aspects of the four components defined within the CP and confirm if from their businesses perspective each is applicable.

  - The Proposer noted that this suggestion that Embedded Generators can potentially reduce the level of network rates is based on the level of assets required and takes into consideration the total expenditure that Ofgem has identified. It was noted the data shows that total expenditure has been less than planned. It was summarised that demand has fallen on networks and that means there is a reduced level of costs at the moment and network rates are one element of that.

  - The Working Group agreed that they would need to decide to what extent to reflect the credits for generation and whether apply the rate to the MEAV of the assets effectively to Charge 1.

  - ….

  - ….

  - ….

- Solution and Legal Text

  - This change proposes to address the issues identified by amending the calculation of credits for EDCM generators to include the cost avoided in relation to exit charges, direct costs, indirect costs and network rates.

  - The proposed solution is that the credits should be calculated in the same way as the equivalent demand costs are derived, but applied as a credit to eligible embedded generators.

  - The charge elements identified are all derived for demand as capacity based charges, but it would seem appropriate for embedded generation for these charges to be applied as a unit based credit to provide an incentive for the generator to export when the system is under most stress and therefore provide the most benefit.

  - The legal text should be derived by the working group based on the solution agreed, but we have set out below the relevant sections for the application of the demand costs:

## Schedule 17 and 18:

## 9.2 A single charging rate, in p/kW/day is calculated as follows:

## Transmission exit charging rate p/kW/day = 100 / DC * NGET charge / (CDCM system maximum load + total EDCM peak time consumption)

## Where:

## DC is the number of days in the Charging Year.

## NGET charge is the DNO Party’s forecast annual expenditure on transmission connection point charges in £.

CDCM system maximum load is the forecast system simultaneous maximum load from CDCM Connectees (in kW) from CDCM table 2506.

Total EDCM peak time consumption (in kW) calculated by multiplying the Maximum Import Capacity of each Connectee by the forecast peak-time kW divided by forecast maximum kVA of that Connectee (adjusted for losses to transmission and, if necessary, for Connectees connected for part of the Charging Year) and aggregating across all EDCM Customer demand.

9.3	The single p/kW/day charging rate is converted into a p/kVA/day import capacity based charge for each EDCM Connectees as follows:

16.7	Transmission exit charge p/kVA/day = [Transmission exit charging rate in p/kW/day] * [Forecast peak-time kW divided by kVA of that Connectee, adjusted for transmission losses and, if necessary for Connectees connected part of the year]

The contribution rates for network rates, direct costs, indirect costs and residual revenue is converted into a £/year import capacity based contribution and a demand sole use asset MEAV based contribution for each EDCM Connectee.

Import capacity based network rates contribution for each Connectee = TNA * NR rate * import capacity

Import capacity based direct operating costs contribution for each Connectee = TNA * DOC rate * import capacity

Import capacity based indirect costs contribution for each Connectee = TNA * INDOC rate * import capacity

Import capacity based residual revenue contribution for each Connectee = TNA * residual revenue rate * import capacity

Where:

TNA is the total site-specific assets (£/kVA) for that EDCM Connectee.

NR rate is the network rates contribution rate in per cent.

DOC rate is the direct operating costs contribution rate in per cent.

INDOC rate is the indirect costs contribution rate in per cent.

Residual revenue rate is the residual revenue contribution rate in per cent.

Import capacity is the Maximum Import Capacity (adjusted, if necessary, if the Connectee is connected for part of the Charging Year) in kVA for that EDCM Connectee.

- Relevant Objectives

## Assessment Against the DCUSA Objectives

  - The Working Group/Proposer considers that the following DCUSA Objective is better facilitated by DCP 287.

| Impact of the Change Proposal on the Relevant Objectives: |  |
| --- | --- |
| Relevant Objective | Identified impact |
| Charging Objective Two - that compliance by each DNO Party with the Charging Methodologies facilitates competition in the generation and supply of electricity and will not restrict, distort, or prevent competition in the transmission or distribution of electricity or in participation in the operation of an Interconnector (as defined in the Distribution Licences) Charging Objective Three - that compliance by each DNO Party with the Charging Methodologies results in charges which, so far as is reasonably practicable after taking account of implementation costs, reflect the costs incurred, or reasonably expected to be incurred, by the DNO Party in its Distribution Business. | Positive |

  - This Change Proposal better meets charging objective two as the tariffs will be more cost reflective and therefore result in a more efficient dispatch of plant and the siting of plant within the distribution network. Both of these will result in the promotion of effective competition in generation.

  - This Change Proposal better meets charging objective three as it increases the cost reflectivity of tariffs within the EDCM by awarding credits to embedded generators that more closely reflect the benefits they bring to DNOs and thereby encourages the development of efficient, co-ordinated and economical distribution networks.

- Impacts & Other Considerations

#### Does this Change Proposal impact a Significant Code Review (SCR) or other significant industry change projects, if so, how?

  - The Working Group does not consider at this stage, there to be any cross-code impact.

#### Consumer Impacts

## The Working Group did not identify any consumer impacts introduced by this change.

#### Environmental Impacts

## In accordance with DCUSA Clause 11.14.6, the Working Group assessed whether there would be a material impact on greenhouse gas emissions if DCP 287 were implemented. The Working Group did not identify any material impact on greenhouse gas emissions from the implementation of this CP.

#### Engagement with the Authority

## Ofgem has been fully engaged throughout the development of DCP 287 as an Observing member of the Working Group.

- Implementation

## The proposed implementation date for DCP 287 is April 2019.

- Consultation Questions

## The Working Group is seeking industry views on the following consultation questions:

| Number | Questions |
| --- | --- |
|  | - Do you understand the intent of DCP 287? |
|  | - Are you supportive of the principles of DCP 287? |
|  | - Do you have any comments on the proposed legal text? |
|  | - Do you consider that the proposal better facilitates the DCUSA Charging Objectives? Please give supporting reasons. |
|  | - Are you supportive of the proposed implementation date of 1 April 2019? |
|  | Do you have any other comments on DCP 287 |
|  | Are you aware of any wider industry developments that may impact upon or be impacted by this CP? |
|  | - Are there any alternative solutions or unintended consequences that should be considered by the Working Group? |

## Responses should be submitted using Attachment 1 to dcusa@electralink.co.uk no later than, xx xxxx 2017.

## Responses, or any part thereof, can be provided in confidence. Parties are asked to clearly indicate any parts of a response that are to be treated confidentially.

#### Attachments

- Attachment 1 – DCP 287 Voting Form

- Attachment 2 – DCP 287 Draft Legal Text

- Attachment 3 – DCP 287 Change Proposal