| DCUSA Consultation |  | At what stage is this document in the process? |
| --- | --- | --- |
| DCP 283 The Calculation of Generation Credits in the CDCM Raised as a Standard Change on 12 October 2016 |  | 01 – Change Proposal. 02 – Consultation. 03 – Change Report. 04 – Change Declaration. |
| Purpose of Change Proposal: DCP 283 seeks to amend the calculation of credits for embedded generation to more closely reflect the benefits they bring to Distribution Network Operators. This document is a Consultation issued to DCUSA Parties and any other interested Parties in accordance with Clause 11.14 of the DCUSA seeking industry views on DCP 283. |  |  |
|  | The Workgroup recommends that this Change Proposal should proceed to a second Consultation. Parties are invited to consider the questions set in section 10 and submit comments using the form attached as Attachment 1 to dcusa@electralink.co.uk by xx Month 2017 The Working Group will consider the consultation responses and determine the appropriate next steps for the progression of the Change Proposal (CP). |  |
|  | Impacted Parties: Distribution Network Operators (DNOs), Generators, Suppliers |  |
|  | Impacted Clauses: Schedule 16 (CDCM) |  |

Guidance On The Use Of This Template:

Code Administrators will produce this Report using the original proposal as the source.

The Workgroup will verify all of the information provided, adding the Impact Assessment.

| Contents 1. Summary 3 2 Governance 3 3 Why Change? 4 4 Code Specific Matters 6 5 Working Group Assessment 6 6 Relevant Objectives 15 7 Impacts & Other Considerations 16 8 Implementation 16 9 Solution and Legal Text 16 10 Consultation Questions 17 Timetable The timetable for the progression of the CP is as follows: #### Change Proposal timetable. #### Activity; #### Date. Initial Assessment Report Approved by Panel; 19 October 2016. First Consultation issued to Parties; 14 March 2017. Second Consultation issued to Parties; xx Month 2017. Change Report issued to Panel; 13 September 2017. Change Report issued for Voting; 22 September 2017. Party Voting Ends; 13 October 2017. Change Declaration issued to Authority; 17 October 2017. Authority Decision; 21 November 2017. Proposed Implementation Date; 01 April 2019. | Any questions? |
| --- | --- |
|  | Contact: Dylan Townsend |
|  | email address DCUSA@electralink.co.uk |
|  | telephone 020 7432 2859 |
|  | Proposer: Johannes Nowak |
|  | email address johannes.nowak@ mvv.de |
|  | telephone |

- Summary

#### What?

## The Distribution Connection and Use of System Agreement (DCUSA) is a multi-party contract between electricity Distributors and electricity Suppliers and large Generators. Parties to the DCUSA can raise Change Proposals (CPs) to amend the Agreement with the consent of other Parties and (where applicable) the Authority.

#### Why?

## DCP 283 has been raised by MVV Environment Services Limited and suggests two changes that could improve the cost reflectivity of generation credits for embedded generators. More cost reflective credits for generators will place incentives on embedded generation that reflect the benefits they bring to network operators.

#### How?

## The current arrangements award credits to embedded generators at all voltage levels above but not including the voltage of connection. This proposed solution is to:

## exclude the customer contributions discount in the assessment of credits for embedded generators in the Common Distribution Charging Methodology (CDCM);

## award a partial credit at the voltage of connection for Low Voltage (LV) connected non-intermittent generation based on a proposed share of asset costs avoided; and

## award a full credit at the voltage of connection for Low Voltage Substation (LVS) connected non-intermittent generation.

- Governance

#### Justification for Part 1 Matter

## DCP 283 is classified as a Part 1 matter and therefore will go to the Authority for determination after the voting process has completed.

## This issue is considered a Part 1 Matter as it affects the level of charges for embedded generators and therefore impacts on competition for embedded generation as specified under DCUSA clause 9.4.2 (A).

#### Requested Next Steps

## Following a review of the Consultation responses, the Working Group will work to agree the detail of the solution for DCP 283.

- Why Change?

#### Background of DCP 283

## The Proposer has raised this CP to address two issues with the calculation of credits within the CDCM; the discounting of credits to take account of customer contributions the principle of applying credits at the voltage of connection. These issues are considered separately below:

## Treatment of customer contributions

## Within the CDCM, demand charges are reduced by customer contributions to take account of amounts paid up front when customers connect. Customer contributions for demand are also applied to the calculation of generation credits. The impact of the application of customer contributions is to reduce the level of credits.

## It is the Proposer’s view that when a generator connects to the network, one of the benefits that is realised by the DNO is a reduced flow on the local network. This allows further demand customers to connect without the need for reinforcement and therefore they will need to make less or no customer contribution when they connect. Consequently, applying the customer contributions to generation credits reduces the cost reflectiveness of the credit that is provided to embedded generation under the CDCM.

## Credits at the voltage of connection

## The principle applied within the CDCM is that credits are awarded to embedded generators for voltage levels above but not including the voltage level of connection. For demand, costs are taken into account down to and including the voltage of connection. The rationale for awarding credits above the voltage level of connection was set down when the CDCM was developed and was justified as the benefit of reduced reinforcement was perceived to be higher up the network. The requirement was set out in an Ofgem decision document in 2008 within Appendix 2 which outlines the principles and assumptions to be used when setting out the CDCM. The relevant assumption is set out in 1.51 which states:

## “1.51. The network is assumed to be demand dominated. Credit will be provided for offsetting demand on the distribution network above the voltage of connection “

## The Ofgem decision is based on Engineering Recommendation P2/6 as supported by ETR 130 Application Guide for Assessing the Capacity of Networks Containing Distributed Generation and applies to both intermittent and non-intermittent generation.

## The basic principle of P2/6 and ETR130 is that embedded generation can offset the need for network capacity depending on the reliability of the generator and its setup. A simple example where an embedded generator offsets the need for a transformer is shown in the diagram below:

## The more reliable the generator is the more the DNO can rely on it for network planning purposes. P2/6 sets out the reliability factors (labelled “f” factors) for different types of generation. Where a generator is intermittent, an additional persistence factor is also taken into account.

## When assessing the ability of an embedded generator to offset network capacity, P2/6 refers to a demand group. The demand group is not specified at a network level and the assumption within the CDCM is that the benefit will be realised at the next voltage level up (e.g. for embedded generation connected to the LV circuit level, the benefit will be realised at the LVS transformer).

## High Voltage

## At High Voltage (HV), DNOs typically exclude HV connected generators when considering the network required to meet the demand for a new customer. However, at the substation, they take account of any embedded generation and consequently less capacity may be required at the substation and voltage levels above. This principle suggests that the current principle within the CDCM of awarding credits for the voltage levels above the voltage of connection is correct as the benefit to the DNO is only realised at higher voltage levels. The Proposer is not suggesting an amendment to the methodology for credits for HV connected generators.

## Low Voltage Substation

## Embedded generators who connect directly at LVS do not currently get a credit for avoiding the use of the LV substation. However, the principle that the benefit is realised at the substation where the capacity can be reduced holds true even though the generator is connected directly to a substation, provided other customers are also connected to that substation and so the power output from the generator can flow to demand customers without using the HV/LV transformer. It is therefore appropriate that, if LVS generators are predominantly at shared substations, LVS generators should receive the benefit at the voltage of connection. However, as the generator will only benefit the DNO if it can be relied on, the Proposer is suggesting extending the credits to the voltage of connection for non-intermittent generation only.

## Low Voltage

## Embedded generators connected to the LV network are not particularly visible to DNOs. When a DNO is planning the LV network, they are more likely to assess the maximum demand at the local substation with some consideration of any large generation that may be connected. At the LV network the presence of generation will be more diverse and therefore the Proposer believes some of the benefits will be realised at the level of connection in addition to the higher voltage levels. The Proposer wishes to take account of generation credits at the voltage of connection for LV connected generation by allocating a proportion of the demand costs at the voltage of connection as a credit to non-intermittent embedded generation at LV. The Proposer suggests a 75% sharing factor for the proportion of the LV demand charge that should be allocated to LV connected generation, but suggest that this value would need further consideration by the Working Group.

- Code Specific Matters

#### Reference Documents

n/a.

- Working Group Assessment

#### DCP 283 Working Group Assessment

## The DCUSA Panel established a Working Group to assess DCP 283. This Working Group consists of DNO, Supplier, National Grid and Ofgem representatives. Meetings were held in open session and the minutes and papers of each meeting are available on the DCUSA website – www.dcusa.co.uk.

#### Request for Information and First Consultation

  - To assist the Working Group in assessing the CP a Request For Information (RFI) was issued to DNOs. The purpose of the RFI was to help establish how DNOs plan their network and the extent to which they rely on embedded generation from a planning perspective. The outcome of the RFI was used to develop the first consultation document.

  - To aid the further develop of the solution for DCP 283, the Working Group issued a consultation to Parties. The responses and subsequent review of responses by the Working Group is detailed within Attachment x and an overview of the conclusions drawn from the review are outlined below.

#### Credits at the voltage of connection

  - The Working Group agreed on the need for a consultation question on whether credits should be awarded at each voltage level of connection.

#### High Voltage

  - The Proposer did not suggest any amendments to the methodology for credits for HV generators and the Working Group agreed with the Proposer to not award credits to HV generators at the voltage of connection. The first consultation sought to confirm if industry agreed with the Working Group and Proposer that credits not be awarded to HV generators at the voltage of connection. The Working Group noted that all respondents agree that there should be no credits made available at the HV connection level and that this is in-line with the view of the Working Groups.

#### Low Voltage Substation

  - The Proposer believed that where a generator connects directly to a LV substation, the saving in reinforcement costs are the same as that achieved by a LV connected generator. The Proposer therefore suggested that it is appropriate that the credit for LVS connected generators should include deferred reinforcement costs at the LVS voltage level as the generator is effectively connecting at LV. Using the RFI data the Working Group concluded that the majority of LVS connected generators have sole use substations and decided it was not appropriate to award credits at the voltage of connection to generators connected at the LVS network level. The Working Group sought views from industry on the question of should LVS connected generators not receive credits at the voltage of connection. The Working Group noted that all respondents agreed that generators should not be awarded credits at the voltage of connection.

#### Low Voltage

  - The Working Group proposed arguments for and against the awarding of credits to LV connected non-intermittent generators at the voltage of connection. Views from industry were sought on whether they believed that credits should be awarded to non-intermittent LV connected generators at the voltage of connection. Respondents views were mixed but the Working Group agreed that more work would be required to proceed with a solution.

  - The Proposer has since raised concerns that continuing to focus on the issue of awarding credits to LV non-intermittent generation at the voltage of connection is likely to result in a delay to the change proposal. Additionally, the Proposer noted concerns that the process is unlikely to yield a sufficiently high degree of evidence to enable this part of the change proposal to progress. Consequently, the Proposer has agreed that it would be sensible for the working group to proceed with just the customer contributions element of this change and the Working Group support this decision.

#### Sharing Factors

  - Under the proposal, the sharing factor represents the proportion of the credit that would be applied at the voltage of connection if the calculation of generation credits was extended to include the voltage level of connection. The sharing factor should represent the amount of upstream assets on the LV network that is offset by a LV generator. As a result of the Proposer and Working Group agreeing to only proceed with

## Treatment of customer contributions

  - The Proposer believes that when a generator connects to the network, one of the benefits that is realised by the DNO is a reduced flow on the local network and at higher voltage levels. The Proposer believes that this allows further demand customers to connect without the need for reinforcement and therefore they will need to make less or no customer contribution when they connect. Consequently, the Proposer believes that applying the customer contributions to generation credits reduces the cost reflectiveness of the credit that is provided to embedded generators under the CDCM.

  - The Working Group discussed that the treatment of customer contributions is a forward looking process. To offset future costs, the Working Group noted that there is a need for credits. However not all members of the Working Group agreed with the Proposer’s assertion that customer contributions will necessarily be reduced by the presence of embedded generation. However, the Proposer’s view is that the benefit would still be realised at higher voltage levels and that this element of the change proposal, on a standalone basis, would only remove customer contributions from the calculation of generator credits at higher voltage levels.

  - The consultation sought views on whether industry believes a cost saving occurs when generation connects which creates a more resilient network and reduces the need for new demand customers to pay contributions. The Working Group noted the need to review the customer contribution section of this proposal to ensure that a case has been made for the treatment of customer contributions and that specific examples are provided for clarity.

## Second Consultation

  - Customer contributions are the amount customers pay when they connect to the distribution network. When customers connect, the DNO will determine what assets need building/ reinforcing and this cost is split between the customer and the DNO. The apportionment rules are set out in the Common Connection Charging Methodology (CCCM) in Schedule 22 of DCUSA.

  - Customers tend to pay most of the cost of connecting at the voltage of connection. At voltage levels above the voltage of connection, the DNO pays a greater proportion. This is to ensure connection charges are fair (eg a domestic customer shouldn’t have to pay for an upgrade to a primary substation). An example of the amount customers pay can be seen in the values used in the CDCM. Below is an extract from the ENWL April 2018 customer contributions input table (132/HV assets removed for clarity):

|  | Assets 132kV | Assets 132kV/EHV | Assets EHV | Assets EHV/HV | Assets HV | Assets HV/LV | Assets LV circuits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LV network |  |  |  |  | 30.00% | 30.00% | 97.00% |
| LV substation |  |  |  |  | 30.00% | 97.00% |  |
| HV network |  |  |  | 57.00% | 91.00% |  |  |
| HV substation |  |  | 57.00% | 92.00% |  |  |  |

  - Using this data, an average ENWL customer that connects to the LV network, can expect to contribute 97% towards any costs associated with the LV network and 30% towards any costs incurred at the HV/LV substation and 30% at the HV network level.

  - Within the CDCM, the 500MW costs form the basis of the asset costs element of the unit rate charges for each tariff. The unit rate yardstick charge in respect of asset costs (not opex) are discounted to take account of the customer contributions already made. So, for a LV connected customer in ENWL’s area, the asset costs that form part of their unit rate are discounted by 97% for the LV network level, and 30% for the HV/LV and HV network levels.

  - This is a reasonable approach, as demand customers should not pay for the same asset twice and any revenue shortfall is picked up through scaling.

  - It should be noted that the 500MW model represents shared use assets and the service models represent sole use assets as referenced in the CDCM user manual. This is an important distinction as the customer contribution discounts are applied to the 500MW model and therefore to the shared use assets, not sole use assets.

  - The cost of connecting new customers can be split into two cost elements as follows:

- Extension assets – “are assets installed to connect a party or parties to the existing distribution network but which exclude Reinforcement assets”

- Reinforcement assets – “Reinforcement is defined as assets installed that add capacity (network or fault level) to the existing shared use Distribution System”

  - An example connection and the associated costs is contained below. This is for a new connection of an 8MVA industrial premises at HV (example 12 from the CCCM):

  - The costs associated for this connection are shown below. The customer contributes £500k towards the total reinforcement cost of £1,500k, which equates to c33% and the full extension asset costs of £130k.

## Extension Assets

  - Extension asset costs are the costs associated with connecting a site to the shared use network. For example, when a housing estate connects with its own LV substation, all the new assets are defined as extension assets. However, once connected, some of the extension assets can be defined as sole use (ie those connecting an individual dwelling) and some as shared use.

  - The existing CDCM methodology reduces asset costs by the level of customer contributions made by demand customers. However, it is possible for embedded generation to reduce extension asset costs in the same way as if they are connected to the existing shared use network. Where a new connection is developed that incorporates embedded generation, the extension assets required may be reduced. This may be through embedded generation connected directly to individual premises or directly to the new distribution network. In addition, once the extension assets are absorbed into the shared use network, the connection of an embedded generator potentially enables future demand connections without more infrastructure investment. Consequently, the embedded generator can be viewed as providing an incremental benefit to the network it is connected to and it is appropriate that that network should be valued on a standalone basis (ie not after removing customer contributions)

## Reinforcement Assets

  - Generation credits are based on the negative demand costs at voltage levels above the level of connection. Consequently, the asset costs that form part of the demand charge (and therefore the credit to generation) are reduced to take account of customer contributions.

  - DCP 283 raises the issue of whether it is appropriate to discount generation credits for customer contributions in the same way as demand customers. The CDCM is a forward-looking model and connecting generation can enable future demand customers to connect without the customer or DNO incurring significant reinforcement.

  - In the example above, a generator directly connected into the primary substation or lower down the network and offsetting the load at the primary substation, potentially removes the need for reinforcement. The saving achieved is the total reinforcement cost, not the reinforcement cost less the customer contribution. In this example, it would therefore be appropriate to determine the generation credits based on the full cost before the customer contribution is deducted.

## Conclusion

  - The 500MW model represents the shared use assets on the distribution network. Embedded generation connects to the shared use network and therefore reduces costs associated with the 500MW model. Customer contributions reduce the value of the 500MW model when it is moved to a unit basis and applies it to both demand and generation. This may be appropriate for demand, but for generation it reduces the underlying asset costs that the embedded generator can offset. This holds true for offsetting both reinforcement costs on the existing network and also for extension assets where savings potentially accrue when embedded generation is incorporated into the design. In addition, once extension assets become part of the shared use network, new embedded generation connecting into this part of the network enables future demand customers to connect at lower cost.

## Other Considerations

## DCP 243 ‘Treatment of Customer Contributions in the CDCM’

  - The Working Group agreed that it was worth monitoring DCP 243 as it is seeking to standardise customer contributions and one of the proposed solutions is to set customer contributions to zero which removes the issue of the treatment of customer contributions. The DCP 243 Working Group have issued an RFI to DNO Parties’. They are due to meet on 21 March 2017 to review RFI responses and agree preferred solutions to take forward to consultation. The DCP 243 Working Group are aiming to deliver the Change Report to the DCUSA Panel in June.

- Relevant Objectives

## Assessment Against the DCUSA Objectives

## The Proposer considers that the following DCUSA Objectives are better facilitated by DCP 283.

| Impact of the Change Proposal on the Relevant Objectives: |  |
| --- | --- |
| Relevant Objective | Identified impact |
| Charging Objective Two - that compliance by each DNO Party with the Charging Methodologies facilitates competition in the generation and supply of electricity and will not restrict, distort, or prevent competition in the transmission or distribution of electricity or in participation in the operation of an Interconnector (as defined in the Distribution Licences) | Positive |
| Charging Objective Three - that compliance by each DNO Party with the Charging Methodologies results in charges which, so far as is reasonably practicable after taking account of implementation costs, reflect the costs incurred, or reasonably expected to be incurred, by the DNO Party in its Distribution Business. | Positive |

## The Proposer believes that Charging Objective Two is better facilitated by DCP 283 because more cost reflective tariffs will provide a more accurate price signal which will result in a more efficient dispatch of plant and the siting of plant within the distribution network. Both of these will result in the promotion of effective competition in generation.

## The Proposer believes that Charging Objective Three is better facilitated by DCP 283 because it increases the cost reflectivity of tariffs within the CDCM by awarding credits to embedded generators that more closely reflect the benefits they bring to DNOs and thereby encourages the development of efficient, co-ordinated and economical distribution networks.

Question x Do you consider that the proposal better facilitates the DCUSA Charging Objectives? Please give supporting reasons.

- Impacts & Other Considerations

#### Does this Change Proposal impact a Significant Code Review (SCR) or other significant industry change projects, if so, how?

## The Working Group does not consider at this stage, there to be any cross-code impact.

#### Consumer Impacts

## Consumer impacts will be assessed following feedback from parties. There may be multiple solutions which may potentially increase the level of credits to embedded generators which could result in an increase cost to demand customers.

#### Environmental Impacts

## In accordance with DCUSA Clause 11.14.6, the Working Group assessed whether there would be a material impact on greenhouse gas emissions if DCP 283 were implemented. The Working Group did not identify any material impact on greenhouse gas emissions from the implementation of this CP.

#### Engagement with the Authority

## Ofgem has been fully engaged throughout the development of DCP 283 as an Observing member of the Working Group.

- Implementation

## The proposed implementation date for DCP 283 is 01/04/2019. Respondents are invited to consider whether they require any further lead time to comply with this change.

Question x Are you supportive of the proposed implementation date of 1 April 2019?

- Solution and Legal Text

#### Credits at the voltage of connection

31. 	In the case of generators, the proportions relate to the notional assets whose construction or expansion might be avoided due to the generator’s offsetting of demand on the network and is set at 0%.

- Consultation Questions

## The Working Group is seeking industry views on the following consultation questions:

| Number | Questions |
| --- | --- |
|  | Do you consider that the proposal better facilitates the DCUSA Charging Objectives? Please give supporting reasons. |
|  | Are you supportive of the proposed implementation date of 1 April 2019? |
|  | Do you have any other comments on DCP 283? |
|  | Are you aware of any wider industry developments that may impact upon or be impacted by this CP? |
|  | Are there any alternative solutions or unintended consequences that should be considered by the Working Group? |

## Responses should be submitted using Attachment 1 to dcusa@electralink.co.uk no later than xx month 2017.

## Responses, or any part thereof, can be provided in confidence. Parties are asked to clearly indicate any parts of a response that are to be treated confidentially.

#### Attachments

Attachment 1 - Consultation Response Form

Attachment 2 - DCP 283 Change Proposal

Ofgem decision document - Delivering the electricity distribution structure of charges