# DCP 283 Draft Legal Text

Schedule 16

- 62. 		For the purposes of this calculation, a full negative contribution to load at all network levels above its Entry Point. At the network level corresponding to circuits a generator’s entry point, the following negative contribution to load is taken into account:

- For all HV connected generation, the contribution is deemed to be zero

- For all LVS intermittent connected generation, the contribution is deemed to be zero and for all LVS non-intermittent connected generation, the contribution is deemed to be zero

- For all LV intermittent connected generation, the contribution is deemed to be zero and for all LV non-intermittent connected generation, the contribution is deemed to be 75%

- For demand users, account is taken of differences between the diversity allowance in the network model and the diversity of each customer group in order to ensure that the estimated load matches the volumes subject to charges in respect of each network level.

- 71. 	For generation users and portfolio tariffs for generation users, a negative contribution to the unit rate (i.e. a credit) comes from each network level above the Entry Point. At the network level corresponding to circuits a generator’s entry point, the following negative contribution to load is taken into account:

- For all HV connected generation, the contribution is deemed to be zero

- For all LVS intermittent connected generation, the contribution is deemed to be zero and for all LVS non-intermittent connected generation, the contribution is deemed to be zero

- For all LV intermittent connected generation, the contribution is deemed to be zero and for all LV non-intermittent connected generation, the contribution is deemed to be 75%

- That contribution is calculated as follows:

[p/kWh from network model assets] = –100*[network level £/kW/year]*[user loss factor]/[network level loss factor]*(1 – [contribution proportion])/[days in year]/24

[p/kWh from operations] = –100*[ transmission exit or other expenditure £/kW/year]*[user loss factor]/[network level loss factor]/[days in year]/24

- 31. 	In the case of generators, the proportions relate to the notional assets whose construction or expansion might be avoided due to the generator’s offsetting of demand on the network is set at 0%.