DCP 179 April 2014 CDCM model r6565

- by Franck Latrémolière on Friday 25 April 2014

This document describes a CDCM tariff model produced to implement a specification issued by the DCP 179 working group on 23 April 2014.

The reference version is model 102 published by the DCUSA Panel in 2013.

## Structural changes

The sheet “NHH” has been renamed “AggCap” to reflect the fact that it now applies to both half hourly and non half hourly tariffs that are billed on an aggregated basis.

The tariff list has changed:

LV Network Domestic and LV Network Non-Domestic Non-CT have been added as aggregated tariffs with red/amber/green unit rates and no capacity or reactive power charges.

The non half hourly LV generation tariff has been relabelled LV Generation NHH or Aggregate HH.

The corresponding LDNO tariffs have been amended accordingly.

## Additional input data

In table 1061. Average split of rate 1 units by distribution time band, additional lines have been added for the two unrestricted tariffs, Domestic Unrestricted and Small Non Domestic Unrestricted.  In the previous model, these tariffs were calculated purely by reference to coincidence and load factors, and this breakdown was not needed.  It is now needed to calculate equalisation factors for the new aggregated half hourly tariffs.

Other input data tables have been modified to reflect changes to the tariff list.

## Additional or modified outputs

There are no changes to the structure of outputs beyond those resulting from the changes to the tariff list.

## New or modified calculation tables

On the Multi sheet:

    - Tables 2408 to 2413 have been replaced with tables 2408 to 2439, which include the implementation of the additional equalisation correction factors.

    - Tables 2425 to 2429 have been replaced with table 2451.  This simplifies the implementation of the DCP 130 rules for unmetered supplies without changing the methodology or the tariffs.

Table 1 summarises the operation of each table on the new Multi sheet.

| Table 1 Load characteristics calculations (Multi) |  |
| --- | --- |
| 2401 | Adjust annual hours by distribution time band to match days in year This adjusts the number of annual hours by distribution time band provided on the Input sheet to ensure that it agrees exactly with the number of days in the charging year. |
| 2402–2406 | Normalisation of split of rate 1/2/3 units by time band; Split of rate 1/2/3 units between distribution time bands These tables combine data for non half hourly metered and half hourly metered tariffs in order to construct a consolidated table of the proportion of consumption under each tariff and unit rate which falls within each distribution time band. Values are normalised to add up to 100 per cent (there might be legitimate differences due to rounding, for example). |
| 2407 | All units (MWh) This calculates the total number of units for each tariff. |
| 2408– 2410 | Calculation of implied load coefficients for one/two/three-rate users These tables calculate implied load coefficients from the distribution of units between time bands, if there were no correlation between user and system peaking within each distribution time band. This assumes that the time of system peak is always in the red time band. |
| 2411 | Calculation of adjusted time band load coefficients This calculates the factor that needs to be applied to reconcile implied load coefficients (from above) with actual load coefficients (from the Loads sheet). |
| 2412–2413 | Normalisation of peaking probabilities; peaking probabilities by network level (reshaped) This table copies the peaking probability data provided by the user, normalising the probabilities so that they add up to 100 per cent where they should, and reshaping it to match the structure of the calculations on the Multi sheet. |
| 2414 | Pseudo load coefficient by time band and network level This is, before the equalisation process, what would be used instead of the coincidence factor to load factor ratio to calculate contributions of each network level to charges within each distribution time band. |
| 2415 | Aggregated half hourly pseudo timeband load coefficients This extracts the pseudo load coefficient by time band and network level relevant to each aggregated half hourly tariff subject to the equalisation process. |
| 2416–2421 | Off-peak non half hourly units (MWh); Off-peak non half hourly timeband use; Multi rate non half hourly units (MWh); Single rate non half hourly timeband use; Single rate non half hourly units (MWh); Multi rate non half hourly timeband use These tables extract data for each non half hourly off-peak tariff, each non half hourly two-rate tariff and each non half hourly unrestricted tariff subject to the equalisation process. |
| 2422 | Average non half hourly timeband use This calculates the deemed use of the red amber and green timebands by consumption across each category of non half hourly tariffs subject to the equalisation process. |
| 2423 | Aggregated half hourly tariff pseudo load coefficient using average non half hourly unit mix This applies the pseudo load coefficients from table 2415 to the consumption pattern from table 2422. The result reflects the network capacity and charges that would be attributed to the non half hourly average load if it was on the aggregated half hourly tariff before equalisation. |
| 2424–2428 | Off-peak non half hourly pseudo timeband load coefficients; Off-peak non half hourly tariff pseudo load coefficient; Single rate non half hourly tariff load coefficient; Multi rate non half hourly pseudo timeband load coefficients; Multi rate non half hourly tariff pseudo load coefficient These tables calculate the network capacity and charges that would be attributed to relevant non half hourly loads before equalisation. |
| 2429 | Average non half hourly tariff pseudo load coefficient This calculates the average network capacity and charges that would be attributed, on average, to non half hourly average loads on the non half hourly tariffs before equalisation. |
| 2430 | Relative correction factor for aggregated half hourly tariff These are the main equalisation factors, raising or lowering the aggregated half hourly tariff relative to non half hourly tariffs in order to achieve the desired tariff equalisation. |
| 2431–2433 | Aggregated half hourly units (MWh); Aggregated half hourly timeband use; Aggregated half hourly tariff pseudo load coefficient These tables calculate the network capacity and charges that would be attributed to relevant aggregated half hourly loads before equalisation. |
| 2434 | Correction factor for non half hourly tariffs These additional correction factors apply to all relevant tariffs on top of the relative correction factor. Their purpose is to ensure that equalisation has no net effect on aggregate network use across the relevant tariffs. |
| 2435–2438 | Single rate non half hourly corrected pseudo timeband load coefficient; Multi rate non half hourly corrected pseudo timeband load coefficient; Off-peak non half hourly corrected pseudo timeband load coefficient; Aggregated half hourly corrected pseudo timeband load coefficient These tables apply the correction factors to each tariff to achieve equalisation. |
| 2439 | Pseudo load coefficient by time band and network level (equalised) This table brings together the results of the equalisation process with the (unmodified) data for tariffs not subject to equalisation. |
| 2440–2442 | Unit rate 1–3 pseudo load coefficient by network level This is what needs to be used instead of the coincidence factor to load factor ratio to calculate each unit rate (excluding unmetered tariffs). |
| 2443 | Adjust annual hours by special distribution time band to match days in year This adjusts the number of annual hours by special distribution time band provided in table 1066 on the Input sheet to ensure that it agrees with the number of days in the charging year. |
| 2444–2447 | Normalisation of split of rate 1 units by special time band; Split of rate 1–3 units between special distribution time bands These tables combine data for non half hourly unmetered and pseudo half hourly unmetered tariffs in order to construct a consolidated table of the proportion of consumption under each tariff and unit rate which falls within each special distribution time band. Values are normalised to add up to 100 per cent where appropriate. |
| 2448–2449 | Calculation of implied special load coefficients for one/three-rate users These tables calculate implied load coefficients from the distribution of units between special distribution time bands, if there were no correlation between user and system peaking within each distribution time band. This assumes that the time of system peak is always in the red time band. Separate calculations are done for the relevant single-rate tariffs (non half hourly unmetered) and for the relevant three-rate tariffs (pseudo half hourly unmetered). |
| 2450 | Estimated contributions to peak demand The first column of this table combines data from tables 2422 and 2423. The second column uses these data to calculate the contribution of each unmetered user category to load during the black time band. The third column uses load factor and coincidence factor data to calculate the contribution of each unmetered user category to peak-time demand (where peak time is the coincidence factor concept). |
| 2451 | Load coefficient correction factor for the group This calculates the correction factor for unmetered tariffs, as a single number based on the aggregation of data for all unmetered tariffs, in line with paragraph 72(d) of the methodology. |
| 2452 | Calculation of special peaking probabilities This table calculates black and yellow peaking probabilities, either by inferring them from the data in table 1069 (if provided) or by applying a rule that the peaking probability per hour should be the same in yellow as in amber. |
| 2453–2454 | Special peaking probabilities by network level; Special peaking probabilities by network level (reshaped) These tables bring together and reshape the black, yellow and green peaking probabilities into a form suitable for use in subsequent calculations. |
| 2455 | Pseudo load coefficient by time band and network level This is what would be used in the case of unmetered supply tariffs instead of the coincidence factor to load factor ratio to calculate contributions of each network level to charges within each special distribution time band. |
| 2456–2458 | Unit rate 1–3 pseudo load coefficient by network level (special) This is what needs to be used in the case of unmetered supply tariffs instead of the coincidence factor to load factor ratio to calculate each unit rate. |
| 2459–2461 | Unit rate 1–3 pseudo load coefficient by network level (combined) This combines data from tables above in order to compile a table providing for each tariff to which the rule at paragraph 72 of the methodology applies, whether metered or unmetered, the numbers that needs to be used in the case of unmetered supply tariffs instead of the coincidence factor to load factor ratio to calculate unit rates. |

Tables 3101 to 3106 in the NHH sheet have been replaced with tables 3101 to 3107 on the new AggCap sheet.  The new tables operate as described in table 2.

| Table 2 Fixed charges for capacity costs for aggregated tariffs calculations (AggCap) |  |
| --- | --- |
| 3101 | Mapping of tariffs to tariff groups This mapping table defines the tariff groupings used to calculate fixed charges based on capacity costs: LV domestic and small non-domestic tariffs, LV medium non-domestic tariffs, LV substation non half hourly tariffs, HV network non half hourly tariffs. |
| 3102 | Capacity use for tariffs charged for capacity on an exit point basis This estimates the capacity used by each relevant tariff using the load factors from table 1041. |
| 3103 | Aggregate number of users charged for capacity on an exit point basis This calculates the number of exit points (i.e. the number of fixed charges) for each of the table 3101 groups. |
| 3104 | Aggregate capacity (kW) This calculates aggregate table 3102 capacity for each of the table 3101 groups. |
| 3105 | Average maximum kVA by exit point Using tables 3103 and 3104 and an assumed power factor from table 1010, this calculates the notional capacity to be charged through fixed charges for each table 3101 group. |
| 3106 | Deemed average maximum kVA for each tariff This reallocates the group-level data from table 3105 to individual tariffs, using the table 3101 mapping. |
| 3107 | Capacity-driven fixed charge elements from standing charges factors p/MPAN/day This shows the contribution to fixed charges for each relevant tariff from capacity costs at each network level. |

Many other calculation tables have been modified to reflect changes to the tariff list and to update references to tables in the Multi and AggCap sheets.

## Other changes

The other changes are:

    - the implementation of DCP 163 (removal of HV Sub tariffs); and

    - the correction of a few cosmetic defects in model 102.

The most significant cosmetic changes are to table 1001. CDCM target revenue (containing the DCP 066 analysis of target CDCM revenue), which is now incorporated within the Input sheet.  Inputs and calculations are clearly separated into different columns, and the calculations made within the Input sheet are purely illustrative and are not relied upon elsewhere in the model.