DCP 138 Draft Legal Text

Implementation of alternative network use factor (NUF) calculation method in EDCM

### Amend Schedules 17 and 18, Paragraphs 18.7 and 18.8 as follows:

18.7	The caps and collars in the table above were fixed for three years, and were used to calculate charges for the Charging Years 2012/2013 and 2013/2014. The caps and collars are to be re-calculated for the subsequent Charging Years using the schedule described in paragraph 18.8. The network use factors are calculated in accordance with paragraphs 29 and 30 below.

18.8 	Table 7 below sets out the schedule for the calculation of the network use factor (NUF) caps and collars for each Charging Year.

Table 7 NUF cap and collar calculation timeline

|  | Charging Year | NUFs used create the cap and collar |
| --- | --- | --- |
|  | 2011/2012 Submission | 2011/2012 NUFs |
|  | 2012/2013 | 2011/2012 NUFs |
|  | 2013/2014 | 2011/2012 NUFs |
|  | 2014/2015 | Average of 2011/2012, 2012/2013, 2013/2014 NUFs |
|  | 2015/2016 | Average of 2011/2012, 2012/2013, 2013/2014 NUFs |
|  | 2016/2017 | Average of 2011/2012, 2012/2013, 2013/2014 NUFs |
|  | 2017/2018 | 2015/2016 NUFs |
|  | 2018/2019 | Average of 2015/2016, 2016/2017 NUFs |
|  | 2019/2020 | Average of 2015/2016, 2016/2017, 2017/18 NUFs |
|  | 2020/2021 | Average of 2015/2016, 2016/2017, 2017/18 NUFs |
|  | 2021/2022 | Average of 2015/2016, 2016/2017, 2017/18NUFs |
|  | 2022/2023 | Average of 2018/2019, 2019/2020, 2020/2021 NUFs |
|  | 2023/2024 | Average of 2018/2019, 2019/2020, 2020/2021 NUFs |
|  | 2024/2025 | Average of 2018/2019, 2019/2020, 2020/2021 NUFs |

### Amend Schedules 17 and 18, Paragraph 30.6 as follows:

Step 4:

30.6 	Each nodal demand’s proportionate usage of a Branch is determined  using the equation below:

Alloc (£/year) = ([MW usage] / [Total MW usage]) * (Abs [Max contingency flow] / [Rating]) * AMEAV

If the Branch is “generation-dominated”, or (2 * Abs [Base flow load]) ≤ Abs ([Base flow] - [Base flow load]), then use:

Alloc (£/year) = ([MW usage] / [Total MW usage]) * (Abs [Max contingency flow] / [Rating]) * Abs ([Base flow load] / [Base flow]) * AMEAV

Where:

- Alloc 	is the allocation of the AMEAV of the asset to a demand user in £/year

- MW 	usage is the absolute value of the “MW usage” of the asset attributable to that demand user (expressed in MW)

- Total MW usage is the sum of the absolute values of the “MW usage” of all demand users of that asset (expressed in MW)

- Max contingency flow is the maximum post-contingent flow through the asset in MVA. The maximum post-contingency asset flows may be extracted from the ‘locational’ analyses.

- Rating 	is the unadjusted rated capacity of the asset in MVA

- Base flow load is the algebraic sum of power flows through the Branch due to demand only in MW.

- Base flow is the aggregate power flow through the Branch under normal network operation in MW.

- AMEAV is the annualised modern equivalent asset value in £/year of that asset.

- The ratio ([Max contingency flow] / [Rating]) is called the asset utilisation factor and it is capped at 1.

- The quantity (Abs [Max contingency flow] / [Rating]) * Abs ([Base flow load] / [Base flow]) is called the load utilisation factor.

Wragge Lawrence Graham & Co LLP

17 June 2015