Excess Capacity Test Model (ECTM)

About

The ECTM is a demonstration model designed to illustrate an alternative methodology for reflecting excess capacity costs in the CDCM, it is not a final model intended to produce final results, and still requires further development.

Intention

Under the current CDCM, tariffs are structured in three ways.  Very crudely, these tariff structures can be thought of as representing the possibility of avoiding costs in network planning.  Hence, the service cable costs which are unavoidable while a user remains connected to a network irrespective of their load or capacity (within the typical bounds of their service model) are fixed.  Higher voltage network assets where loads are more diversified are considered to be entirely avoidable and are represented by unit rates.  In between these two extremes, capacity charges represent the size of assets required to be reserve at or immediately above the level of connection.  The underlying assumption may be considered to be that local network assets need to be size for the scale of an individual user’s peak demand, because the scope for diversity from other users at the point in the network is limited.

In the CDCM these capacity charges are calculated by assigning network level costs to customer types using fixed standing charge factors which vary according to the type of customer and connection voltage.  This is how the capacity costs are limited to just those immediately local assets, and not those at a higher network level.

Under this alternative approach, for excess capacity charges, new standing charge factors are introduce that apply an excess capacity charge for all assets at all network levels.  Hence, under this approach rather than paying for the higher network assets via unit rates, for the excess capacity, these costs are reflected by a higher excess capacity cost.  The cost reflectivity of the model overall is unaltered but the way in which those costs are reflected in tariffs is adjusted.

Modelling

The example model is based on the ENWL published 25-26 CDCM.  No inputs have been changed, and in testing no model rebalancing (the process of multiple iteration across all charging models) has been undertaken.

Alternative standing charge factors are added to the “standing charge factors” sheet as below.  The factors are set to 100% for all network levels for those customer types with excess capacity charges.

In the “Capacity charges” sheet these factors are fed in and used in the excess capacity calculation which creates a differential charge in comparison to the standard capacity charge.  Example below:

Excess capacity tariffs in this example are around triple those of standard capacity charges for LV Site Specific customers, and around double for HV Site Specific customers.  As the charge is generated by applying higher voltage network asset costs to the charges it is as expected that the charge differential would be lower for higher voltage connected customers.

Economic justification and observations

There is further work to do to consider potential secondary impacts on unit rates which perhaps will need to be adjusted to reflect the degree of assets paid for by excess capacity charges.  This impact will be small as the level of excess capacity is small relative to total capacity.

Potential economic justification may result from data indicating that the nature of network design is different for ‘planned’ for demand within an agreed capacity, and ‘unplanned’ demand resulting from exceedance of agreed capacity.  If networks need to build in a tolerance when sizing network assets that reflects this excess capacity usage, then it may be argued it is more appropriate to reflect those asset costs through the excess capacity charge rather than via unit rates.  If networks size higher network assets under the DRM assumption conditions based only on the net level of observed or expected diversified demand then it may seem more appropriate to retain the status quo of unit rates charges.  This question needs to be explored in more depth.