DCUSA REQUIREMENTS RELATED TO THE IMPLEMENTATION OF ‘P432, HALF HOURLY SETTLEMENT FOR CT ADVANCED METERING SYSTEMS’ & DCP 414

This document sets out various aspects of the solutions developed under P432 and DCP 414 and clarifies the requirements and obligations on DCUSA Parties purposes of ensuring that there is transitional protection for customers affected by BSC Modification P432 or MHHS (being Customers with CT metering which are required to become half-hourly settled by migrating to Measurement Class C or E, or any other CT Metering Points catered for by MHHS which are migrating to become half-hourly settled on a site-specific basis).

Where do I find the requirements and obligations on DCUSA Parties

Clauses 19.12 to 19.16 in Section 2A of the DCUSA contain the provisions related to the actions required of both DNO/IDNO Parties and Supplier Parties for the purpose noted above.

Paragraphs 179 to 183 of Schedule 16 (CDCM) contain the provisions related to the action required of the DNO/IDNO Parties in terms of DUoS charging arrangements, again, for the  purpose noted above.

Clause 12.13B and 12.13C of Schedule 2B ‘National Terms of Connection’ contain the rights customers have in terms of the process undertaken to move them to HH Settlement and the provision of an associated Maximum Import Capacity value .

What actions are required of Parties (detailed)

The table below has been created to set out the actions needed, which Party needs to take the action and is ordered chronologically.

| Action Owner | Action Reference | Action Description |
| --- | --- | --- |
| DNOs/IDNOs | Clause 19.14 | “The Company shall provide contact details to the User relating to agreeing the Maximum Import Capacity.” This means that DNOs/IDNOs are to proactively contact Suppliers to provide the contact details which the DNO/IDNO wants the Supplier to pass on to their customers for the purposes of agreeing the MIC |
| Suppliers | Clause 19.15 | Whilst it is not explicit within the text itself, one requirement for suppliers will of course be to identify any of their customers who will be impacted. Then in accordance with the Clause 19.15, each Supplier with customers who are impacted are to provide each of their impacted customers with the following information (at a minimum): - site address information, including MPAN and Meter Serial Number(s); - reason for the correspondence; - expected migration date; - the relevant DNO/IDNO Party’s contact details (as provided by the relevant DNO/IDNO Party under Clause 19.14 [step 1 above]); and - explanation of the process, including the Company's assessment which may require the Customer to agree a Maximum Import Capacity. |
| Suppliers | Clause 19.14 | “The User shall confirm to the Company the contact details of all the Customers impacted by P432, or any other CT Metering Points catered for by MHHS, used in providing the information to the Customer under Clause 19.15.” Essentially, this means that Suppliers are to provide the relevant DNO/IDNO Parties, with the contact details of any of their impacted customers. The details to be provided are those which the Supplier used in providing information to their impacted customers as per the step above. In completing this step, it would appear that it is not only the customer contact details that need to be provided the relevant DNO/IDNO Parties but also some additional information that will be necessary for the relevant DNO/IDNO Parties to effectively track the sites in question. It therefore seems reasonable for Suppliers to provide the following data items: - Supplier Name (this could be useful where a single person acts on behalf of multiple Supplier parties and wishes to submit a single set of data) - Supplier MPID - MPAN - Meter Serial Number(s) - Expected Migration Date (this will be useful for DNOs/IDNOs to track the start/end dates for sites across multiple Suppliers) - Customer Name - Customer Phone Number (if available) - Customer Email (if available) - Customer Postal Address (if available) Assume that at least one of either Customer Email or Customer Postal Address will be necessary as one of these is likely to be the way in which a Supplier will have contacted their customer as per the requirement to let them know what is going on. A spreadsheet template has been created for use by Supplier Parties in providing the above information to the relevant DNO/IDNO Parties |
| DNOs/IDNOs | Part 4 of Schedule 16 | For any Premises with CT Metering Points notified to them by Supplier Parties as per the above step, and which have been migrated to either Measurement Class C or E for P432 or required to become half-hourly settled for MHHS, the DNOs and IDNOs will need to make sure that the ‘Domestic Aggregated or CT’ tariff, or the ‘Non-Domestic Aggregated or CT’ tariff as applicable is applied. |
| DNOs/IDNOs | Part 4 of Schedule 16 | After a period of 12 months from the date of the first migration of a Premises (and no later than 6 months following that period), the DNOs/IDNOs will need to assess the capacity of each applicable Premises based on metered data over that 12-month period and agree with the customer (using the contact details provided by the Supplier) an appropriate MIC for that Premises. If the DNOs/IDNOs and any customer cannot agree a MIC, the DNOs/IDNOs will set the MIC based on the highest peak import capacity and the customer shall be informed of the new MIC (in accordance with the NTC notice provisions). |
| DNOs/IDNOs | Clause 19.16 | When the assessment under Part 4 of the CDCM has been completed, the DNOs/IDNOs are to inform the Customer, using the contact details provided under Clause 19.14 of the rights the Customer has under the National Terms of Connection. |

What actions are required of Parties (simplified)

| 19.14 The Company shall provide contact details to the User relating to agreeing the Maximum Import Capacity. The User shall confirm to the Company the contact details of all the Customers impacted by P432, or any other CT Metering Points catered for by MHHS, used in providing the information to the Customer under Clause 19.15. 19.15 Prior to each Customer's transition, the User shall (as a minimum) provide the Customer with the following information: 19.15.1 site address information, including MPAN and Meter Serial Number(s); 19.15.2 reason for the correspondence; 19.15.3 expected migration date; 19.15.4 the Company’s contact details (as provided by the Company under Clause 19.14); and 19.15.5 explanation of the process, including the Company's assessment which may require the Customer to agree a Maximum Import Capacity. 19.16 When the assessment under Part 4 of the CDCM has been completed, the Company shall inform the Customer, using the contact details provided under Clause 19.14 of the rights the Customer has under the National Terms of Connection. | 1) Proactive Communication by Network Operators: DNOs and IDNOs should proactively contact Suppliers to provide the contact details which the DNO/IDNO wants the Supplier to pass on to their customers for the purposes of agreeing the MIC. 2) Identification of Impacted Customers: Suppliers will need to identify which customers are impacted and contact them with the required information. This is to be done prior to transitioning each customer into the new arrangements. 3) Confirmation to Network Operators: Suppliers must confirm to the networks which customers are impacted. This confirmation would likely be done through bilateral communication, such as email and that an agreed format would be useful. It is expected that Suppliers will provide several of the same pieces of information that they provide to their customers, including MPAN, Meter Serial Number(s), expected migration date. 4) 12-Month Assessment Period: Once the migration starts, the networks will begin a 12-month assessment period to determine the maximum import capacity based on the data over that period and will use the contact details provided by the Suppliers, to contact the customers within 6 months of the end of the 12 month assessment period in order to confirm the maximum import capacity that will be applicable to their site and notify them of their rights under the National Terms of Connection (see Clause 12.13B and 12.13C of Schedule 2B. |
| --- | --- |

Legal Text Extracts

SECTION 2A - DISTRIBUTOR TO SUPPLIER/GENERATOR RELATIONSHIPS

Transitional Protection for Customers affected by BSC Modification P432 or MHHS

19.12	Part 4 of the CDCM contains transitional protection for Customers who may be affected by the implementation of BSC modification P432 or any other CT Metering Points catered for by MHHS. All DNO/IDNO Parties shall comply with Part 4 of the CDCM, including a DNO Party operating outside of its Distribution Services Area.

19.13	The User shall initiate the transition for all Customers covered under Clause 19.12. The Company shall conclude the transition for each Customer by allocating them to the correct site-specific tariff under the CDCM such that a Maximum Import Capacity is required (except for those domestic Customers who opt for an aggregated tariff under paragraph 132D of the CDCM).

19.14	The Company shall provide contact details to the User relating to agreeing the Maximum Import Capacity. The User shall confirm to the Company the contact details of all the Customers impacted by P432, or any other CT Metering Points catered for by MHHS, used in providing the information to the Customer under Clause 19.15.

19.15	Prior to each Customer's transition, the User shall (as a minimum) provide the Customer with the following information:

19.15.1		site address information, including MPAN and Meter Serial Number(s);

19.15.2		reason for the correspondence;

19.15.3		expected migration date;

19.15.4	the Company’s contact details (as provided by the Company under Clause 19.14); and

19.15.5	explanation of the process, including the Company's assessment which may require the Customer to agree a Maximum Import Capacity.

19.16	When the assessment under Part 4 of the CDCM has been completed, the Company shall inform the Customer, using the contact details provided under Clause 19.14 of the rights the Customer has under the National Terms of Connection.

SCHEDULE 16 - COMMON DISTRIBUTION CHARGING METHODOLOGY

Part 4 – Transitional Protection for Customers affected by BSC Modification P432 or MHHS

179.	This Part 4 sets out the transitional protection for Customers who may be affected by BSC Modification P432, being Customers with CT metering which are required to become half-hourly settled by migrating to Measurement Class C or E, or any other CT Metering Points catered for by MHHS which are migrating to become half-hourly settled on a site-specific basis.

180.	This Part 4 forms part of the CDCM, but also applies to IDNO Parties and to DNO Parties acting outside of their distribution services area.

181.	Subject to paragraph 183 below, where: (a) a Customer takes a supply of electricity at a Premises where the electricity conveyed to the Premises is recorded through a CT meter; and (b) the Metering Point for such Premises has been migrated to either Measurement Class C or E for P432 or required to become half-hourly settled for MHHS, then, following the date of the migration of the Premises, the Domestic Aggregated or CT tariff, or the Non-Domestic Aggregated or CT tariff as applicable shall be applied.

182.	Within 6 months following the period of 12 months from the date of the first migration of a Premises, the DNO/IDNO Party shall reasonably assess the capacity based on metered data and agree with the customer an appropriate MIC. In the event the DNO/IDNO and customer are unable to agree a MIC, it will be set on the highest peaked import capacity and the customer shall be informed of the new MIC (in accordance with the NTC notice provisions).

183.	In this Part 4, the following definitions shall apply:

BSC Modification P432 means the modification to the BSC referred to as modification ‘P432, Half Hourly Settlement for CT Advanced Metering Systems’, which was approved by the Authority on 15 January 2024.

SCHEDULE 2B - NATIONAL TERMS OF CONNECTION:  SECTION 3:

12	LIMITATION OF CAPACITY

…..

General

12.12	Except where a variation requires a Modification, either Party may propose a variation to the Maximum Import Capacity and/or Maximum Export Capacity by notice in writing to the other Party. The Company and the Customer shall negotiate in good faith such a variation, but where it is not agreed section 23 of the Act may entitle either Party to refer the matter to the Authority.

12.13	Any reduction in the Maximum Import Capacity or the Maximum Export Capacity pursuant to Clause 12.12 shall, where the Parties have within the preceding 12 months agreed the Maximum Import Capacity or the Maximum Export Capacity (as applicable), only take effect following the expiry of 12 months from the date of such previous agreement (unless the Company expressly agrees otherwise).

12.13A	No reduction in the Maximum Import Capacity or the Maximum Export Capacity pursuant to Clause 12.12 shall have effect prior to the first day of the month following the date of the notice properly given under Clause 12.12, stating the required capacity (being, if the reduction initially requested is not agreed, the request for the reduction which is subsequently agreed).

12.13B	Clauses 12.13 and 12.13A shall not apply where Part 4 of Schedule 16 of the DCUSA applies to the Premises.

12.13C	Where no agreement on the Maximum Import Capacity is reached under Part 4 of Schedule 16 of the DCUSA and the Company decides a value based on actual metering data plus headroom, then the amended Maximum Import Capacity shall apply for the purposes of this Agreement and the Company shall have no liability to the Customer should the value chosen not be suitable and/or have an impact on the Connection Assets.