| DCUSA Consultation |  | At what stage is this document in the process? |
| --- | --- | --- |
| DCP 452: Correct application of Forward Cost Pricing EDCM charges to users connected directly to a Grid Supply Point Date Raised: 28 February 2025 Proposer Name: Andrew Enzor Company Name: Field Gaia Ltd Party Category: CVA Registrant |  | 01 – Change Proposal. 02 – Consultation. 03 – Change Report. 04 – Change Declaration. |
| Purpose of Change Proposal: To correct an oversight in DCUSA which results in customers connecting directly to Grid Supply Point incorrectly facing a locational FCP charge relating to assets downstream of their connection. |  |  |
|  | The Working Group recommends that this Change Proposal should: proceed to Consultation Parties are invited to consider the questions set in section 10 and submit comments using the form attached as Attachment 1 to dcusa@electralink.co.uk by XX XXXX XXXX DCP 452 has been designated as a Part 1 Matter and a Standard Change. The Working Group will consider the consultation responses and determine the appropriate next steps for the progression of the Change Proposal (CP). |  |
|  | Impacted Parties: DNOs, IDNOs, Suppliers, CVA Registrants |  |
|  | Impacted Clauses: Paragraphs 6.2 and 6.3 within Schedule 17. |  |

| Contents 1 Summary 3 2 Governance 6 3 Why Change? 6 4 Working Group Assessment 7 5 Legal Text 18 6 Code Specific Matters 19 7 Relevant Objectives 20 8 Impacts & Other Considerations 21 9 Implementation 27 10 Consultation Questions 27 11 Attachments 28 Timetable The timetable for the progression of the CP is as follows: Change Proposal timetable:. #### Activity; #### Date. Initial Assessment Report; 19 March 2025. Consultation Issued to Industry Participants; April/May 2025. Change Report Approved by Panel; 18 June 2025. Change Report issued for Voting; 19 June 2025. Party Voting Closes; 10 July 2025. Change Declaration Issued; 14 July 2025. Authority Decision; 30 September 2025. Implementation; 01 April 2027. | Any questions? |
| --- | --- |
|  | Contact: Code Administrator |
|  | DCUSA@electralink.co.uk |
|  | 02074323000 |
|  | Proposer: Andrew Enzor |

Summary

#### What?

## Under the Forward Cost Pricing (FCP) variant of the Extra High Voltage (EHV) Distribution Charging Methodology (EDCM), Connectees which connect direct to a Grid Supply Point (GSP) are deemed not to use any shared network assets, but are exposed to forward looking charges/credits for the downstream FCP network group in that location. This Change Proposal seeks to remove the non-cost-reflective application of FCP charges/credits to such Connectees.

## There is no equivalent issue under the Long Run Incremental Cost (LRIC) variant of the EDCM. The LRIC approach derives a zero charge for all Connectees which are connected direct to a GSP.

#### Why?

## Connectees at all voltages in the EDCM and CDCM face forward looking charges/credits in respect of assets upstream of their connection point, up to the GSP. Connectees directly connecting to a GSP under the FCP variant of the EDCM are exposed to charges/credits in respect of a downstream voltage, resulting in differential treatment of similar network users, dependant on whether the DNO region uses the FCP variant of the EDCM or the LRIC variant.

## For impacted Connectees, this can have a very material impact. For example, we are aware of a prospective battery storage site in one location which the DNO has indicated will face a non-cost-reflective charge of ~£18/kVA/year for network assets it does not use.

#### How?

## The FCP-related element of the EDCM charge/credit should be set to zero for Connectees which connect direct to a GSP, enacted in either Paragraph 6 of Schedule 17 or Paragraph 2.7 and Paragraph 6.3 of Annex 1 to Schedule 17.

Governance

#### Justification for Part 1 Matter

## DCP 452 is a Part 1 Matter. It will remove charges faced by some users which connect directly to a GSP in DNO regions using the FCP variant of the EDCM, in turn impacting on competition in the generation and supply of electricity.

#### Next Steps

  - Following a review of the Consultation responses, the Working Group will work to agree the final detail of the solution for DCP 452 and if appropriate progress to the Change Report phase.

Why Change?

#### Background of DCP 452

## The EHV Distribution Charging Methodology (EDCM) is intended to give Connectees a locationally-varying forward-looking cost signal related to each user’s impact on the network. This is achieved differently in the Long Run Incremental Cost (LRIC) and Forward Cost Pricing (FCP) variants of the EDCM.

## The FCP method uses the concept of “network groups”. All network assets at a given voltage level which are electrically connected under normal running conditions are assigned to the same network group. All Connectees connected to a given network group face the same FCP charge/credit.

## The LRIC method does not use the concept of network groups. All network assets are treated individually, with each Connectee’s impact on the network assessed based on the impact of incremental demand at their specific location. Connectees face an LRIC charge/credit only in respect of network assets which they directly influence.

## Six DNO licensees use the FCP method (Scottish Hydro Electric Power Distribution plc; Southern Electric Power Distribution plc; SP Distribution Limited; SP Manweb plc; National Grid Electricity Distribution East Midlands plc; and National Grid Electricity Distribution West Midlands plc.). The remaining eight DNO licensees use the LRIC method.

## Under both variants, Sole Use Assets (defined as “assets in which only the consumption or output associated with a single Connectee can directly alter the power flow in the asset“) for each Connectee are separately identified.

## All Connectees charged under both variants of the EDCM are assigned a Point of Common Coupling (PoCC), identifying where their Sole Use Assets connect to the shared network. The PoCC is converted into a Customer Category, encoding the voltages at which a Connectee is deemed to use shared assets.

## Separate charges are calculated in respect of Sole Use Assets, reflecting that those assets are funded by the Connectee in their connection charge. The charge under the EDCM for Sole Use Assets only reflects the costs associated with operation and maintenance. The LRIC and FCP charges/credits only relate to a Connectee’s use of shared networks assets upstream of the PoCC.

Worked Example of FCP Grouping

## Figure 1 shows a small section of network, with a Grid Supply Point (GSP) and two downstream Bulk Supply Points (BSPs). In England and Wales this would typically be a 132kV network; in Scotland this would typically be a 33kV network serving Primary substations in place of BSPs.

Figure 1: Example of FCP Grouping

## The FCP network group for the 132kV network at this GSP would include all assets within the dotted line. A Connectee connecting to any of the circuits within the dotted line would face the FCP charge/credit for the group.

## Conversely under the LRIC approach, a Connectee connecting anywhere faces a charge/credit in respect of any assets on which incremental demand at their location causes change in powerflow. In Figure 1, a Connectee connecting to any of the circuits in the dotted line would impact all of those circuits, so both LRIC and FCP would give a similar, cost-reflective outcome.

Issues with of FCP Grouping for GSP-connected Users at non-interconnected GSPs

## Figure 2 shows the same small section of network, with the addition of Connectee C. Connectee C’s PoCC is at the GSP, so this Connectee would be assigned Customer Category 0000.

Figure 2: FCP Grouping with a GSP-connected Connectee at a non-interconnected GSP

## Under the FCP approach, Connectee C is included within the 132kV network group. The FCP (and LRIC) approaches both only consider thermal reinforcement. In the example shown, Connectee C cannot drive thermal reinforcement on the 132kV circuits to which the FCP charge relates.  But it is (non-cost-reflectively) exposed to the locational charge/credit for the whole network group. Further, the capacity of Connectee C will be added to the denominator of the calculation of the FCP charge, so it will artificially reduce the FCP charge for the network group (on a £/kW basis), distorting charges for all other customers connecting within the network group.

## Connectee C is facing a forward-looking locational charge/credit in respect of thermal reinforcement which it cannot impact.

## Conversely under the LRIC approach, incremental demand at Connectee C’s location is assessed for its impact on network assets. It does not impact powerflow on any of the 132kV circuits, so it correctly sees no locational charge/credit.

FCP Grouping for GSP-connected Users at interconnected GSPs

## Figure 3 shows a similar section of network, but with interconnection between two GSPs. In this instance, the two GSPs are considered as a combined network group in the FCP method.

Figure 3: FCP Grouping with a GSP-connected Connectee at an interconnected GSP

## In this example, Connectee C despite being connected directly at GSP A, can take power from GSP B under normal running arrangements, and so could drive thermal reinforcement on any of the 132kV circuits. In the LRIC variant of the EDCM, the impact of incremental powerflow at Connectee C’s location would be assessed, which could include an impact on the 132kV circuits, in which case it could see a non-zero LRIC charge 1.

The Defect this Change Proposal seeks to address

## Under the FCP variant of the EDCM, Connectees connected directly to a non-interconnected GSP, are incorrectly and non-cost-reflectively exposed to the forward-looking locational charge/credit relating to thermal reinforcement on circuits in the network group associated with the network voltage downstream of the relevant GSP.

The Impact

## If not made, Connectees directly connected to non-interconnected GSPs in DNO areas using the FCP variant of the EDCM will continue to be exposed to forward looking charges/credits non-cost-reflectively in respect of thermal reinforcement which they cannot drive, and will continue to distort forward looking charges/credits for other Connectees within the same network group.

## The working group has assessed the impact…

Question 1- Do you understand the intent of DCP 452?

Question 2: Are you supportive of the principles that support this CP, which are to address what the Proposer believes to be a defect in the FCP variant of the EDCM, for Connectees connected directly to a non-interconnected GSP (Customer Category 0000), where they are incorrectly and non-cost-reflectively exposed to the locational charge/credit for the shared network in the network group associated with the network voltage downstream of the relevant GSP?

Working Group Assessment

#### DCP 452 Working Group Assessment

## The DCUSA Panel established a Working Group to assess and develop DCP 452: Correct application of Forward Cost Pricing EDCM charges to users connected directly to a Grid Supply Point. The Working Group was convened following the DCUSA Panel’s initial assessment, which concluded that the proposal could proceed on an “at risk” basis pending confirmation from Ofgem regarding any potential interaction with the ongoing Distribution Use of System (DUoS) Significant Code Review (SCR). This approval was subsequently received.

## In total, the Working Group has 11 active members representing a cross-section of DNO, IDNO and Generator categories, supported by the Code Administrator and Technical Secretariat.

## The Working Group held four formal meetings between April and June 2025 and developed this consultation document to gather information and feedback from market participants on this CP. During its development of DCP 452, it was agreed by the Working Group that the following items should be addressed in the consultation:

- An assessment of the historical reasons for the two EHV Charging Methodologies and also the differences between FCP and LRIC versions of the EDCMs.

- An assessment of the interaction with the Distribution Use of System (DUoS) Charges Significant Code Review (SCR).

- An assessment of the impacts of the change by way of a Request for Information (RFI) to DNOs to gather real-world data on the number and characteristics of affected customers.

- The provision of updated EDCM models and the inclusion of an overview of the updates that were required to the EDCM models.

- To set out a counter argument to that of the Proposer relating to the design of the FCP methodology and potential impacts of the original solution proposed

- An assessment of DCP-139 Non-Application of FCP charge for Category 0000 Customers and Ofgem’s decision to reject it.

#### Historical reasons and differences of the two EDCM Charging Methodologies

Reasons for Two Methodologies

## When the EDCM was initially developed (around 2010/11), it was believed that LRIC would be universally adopted. However, at that time, some Network companies expressed discomfort with the LRIC approach, preferring an alternative solution, being that of the FCP. It was understood that due to strong views from all involved parties, a single methodology was not taken forward.

Overview of Marginal Charges from FCP and LRIC

## Both FCP and LRIC methods segment the distribution network into various locations to determine marginal charges.

- LRIC locations: A location is typically a node on the EHV network, such as a primary substation or an EHV customer (demand or generator).

- FCP locations: A location is defined as a network group, which can exist at three possible levels:

  - Level 1: Contains 132 kV and similar circuits.

  - Level 2: Contains 132kV/33kV and similar substations, and 33kV and similar circuits.

  - Level 3: Contains primary substations, e.g., 132kV/11kV or 33kV/11kV.

## Each tariff within the EDCM model is associated with either an LRIC or FCP location.

Commonality and Differences in Outputs

## Both methodologies provide similar information for each location:

- Charge 1 (£/kVA/year): Relates to future demand-led reinforcement costs. In LRIC, this is broken down into 'Charge 1 Local' (voltage level of connection) and 'Charge 1 Remote' (voltages above the level of connection). In FCP, this is broken down into the Network Charge 1 (voltage level of connection), Parent Charge 1 (voltage level above the level of connection, if applicable) and Grandparent Charge 1 (voltage level two above the level of connection, if applicable). For both variants, it is expected to drive charges to demand and credits to generation, where generation can defer reinforcement.

- Active (kW) and reactive (kVAr) flows: Provided for generators and demand in maximum and minimum demand scenarios.

## Key differences include:

- In FCP, both generation and demand can exist within the same network group, whereas LRIC requires a node to be modelled as either generation or demand. Consequently, the FCP dataset provides four kW and four kVAr values per location (demand and generation, two scenarios), while LRIC provides two kW and two kVAr values.

- FCP includes a link to a "parent location" (higher-level network group) for each location, establishing a hierarchy of up to three levels. All Charge 1 values throughout this hierarchy are applied to each tariff. Some FCP entries can represent notional "hybrid network groups" for customers supplied from two different network groups.

- LRIC includes links to other nodes supplying the same customer. Negative Charge 1 values are replaced with zero.

Application of FCP/LRIC Charge 1 to Demand Tariffs

## Two components of Charge 1 are applied to demand:

- Super-red rate (p/kWh):

  - For LRIC: The remote Charge 1 (related to voltages above the relevant node) is applied to consumption during a DNO-defined super-red time band. This reflects that network capacity and reinforcement costs are driven by consumption at the network's most loaded times, not individual customer capacity or maximum consumption.

  - For FCP: The parent and grandparent network group Charge 1 is applied during the super-red time band.

- Capacity charge (p/kVA/day):

  - For LRIC: The local Charge 1 is applied to the maximum import capacity. This is based on the premise that there is little diversity within a voltage level, so customer capacity serves as a proxy for required network capacity.

  - For FCP: The network group Charge 1 is applied to the maximum import capacity.

#### Interaction with the Distribution Use of System (DUoS) Charges Significant Code Review (SCR)

## The Working Group noted that the DCUSA Panel, as part of their Initial Assessment of DCP 452, had sought direction from Ofgem as to whether the DCP 452 falls within the scope of the Significant Code Review and therefore, whether it could be progressed at this moment in time. This was in line with the requirements of Clause 10.23 in Section 1C of the DCUSA, which required the Panel to provide their own assessment.

## With respect to their assessment of whether, the proposal falls within the scope of the DUoS Charges SCR, the Panel assessed that to their knowledge, DCP 452 doesn't fall within the known scope of the DuoS SCR and believe it can proceed on this basis.

## As part of the Panel’s assessment of whether the proposal falls within the scope of the Significant Code Review, the Panel noted the representations put forward in section 7 ‘Impacts & Other Considerations’ of the CP form by the Proposer with respect to their CP and the relevance of the DUoS SCR and other ongoing workstreams.

## The Panel also noted that the issue was discussed during the February DCMDG meeting, where an Ofgem representative was in attendance, however, it is understood that no comments from that Ofgem representative have been recorded as to whether they would consider this CP to be within the scope of the SCR.

## On 16 April 2025, Ofgem confirmed the following:

As communicated today at DCUSA Panel and following Panel Chair’s request, I am re-iterating our position in response to DCUSA assessment of impacts on the DUoS SCR in relation to DCP452.

Although we consider DCP452 to be in scope of the DUoS SCR because it relates to the forward looking charges, we are content for DCP452 to proceed in the DCUSA change process.

Due to the niche and discrete nature of the DCP452 issue at this time, it does not feel unreasonable to let industry progress with this CP.

However we would like to caveat this by saying that our upcoming work on the DUoS SCR may lead us to later subsume the CP should we discover that the DCP452 is contrary to the direction of travel of SCR. The proposer, DCUSA Panel and stakeholders should understand this and be content to proceed on this basis.

#### Request for Information (RFI) and Summary of Results

## To support the Working Group’s assessment of DCP 452 and to inform the consultation process, a targeted RFI was issued to DNOs who currently apply the FCP variant of the EDCM. The purpose of the RFI was to gather real-world data on the number of affected customers and the financial impact of the proposed change, thereby enabling a more robust and evidence-based evaluation of the issue.

Development of the RFI

## An RFI template was developed to ensure consistency in data collection and it requested DNOs to extract specific data their populated EDCM models and to submit the output of that extraction as aggregated data, specifically:

- The number of customers classified as Category 0000.

- The number of those customers receiving super-red export credits or incurring Charge 1 capacity charges.

- The aggregate value of those credits and charges.

- The total number of EDCM customers per licence area.

## Instructions were provided to ensure consistent extraction of data from the relevant worksheets within the EDCM model, and DNOs were asked to submit one response per applicable licence area. The deadline for responses was set for 30 April 2025, in advance of the second Working Group meeting.

## The results of the RFI have been included in Section 8 below.

#### Counter Argument of Proposer’s view of issue and solution

Argument 1: Consistency with Network Group Definition in the FCP Approach

Initial Counterargument

## A counter view to what has been set out in the Change Proposal is that the methodology for calculating FCP charges for demand customers, as described in Annex 1 of Schedule 17 (Paragraph 6.5 and Figure 8 added below for ease of reference), defines the GSP busbar (on the distribution side) as part of the Level 1 network group.

## It is interpreted that customers whose point of common coupling is at the GSP busbar (Category 0000) are thus within a Level 1 network group. Consequently, the FCP charges applicable to this network group should apply to these customers to maintain consistency with the established FCP EDCM methodology. Exempting Category 0000 customers from FCP locational charges could introduce inconsistency compared to other customers within the Level 1 network group and those in Level 2/3 network groups.

Proposer's Perspective

## The Proposer suggests that while a 0000 customer may be within the network group, their connection does not drive reinforcement within that network group. Building on the example diagram, a new 0000 customer connecting within a Level 1 network group would not impact power flows on other Level 1 circuits. Therefore, it is considered inappropriate for such a customer to face forward-looking FCP charges related to those circuits.

## Possible alternative solutions are suggested, such as redefining network groups to explicitly exclude 0000 customers from the Level 1 network group. Furthermore, it is noted that Paragraph 2.16 of Schedule 17 effectively calculates the net present value of reinforcement within the network group and divides it by total capacity. The Proposer therefore argues that there is the potential to exclude the capacity of 0000 customers from the total capacity denominator, either through adjustments to the definitions of Cl and D in Paragraph 2.16 or by explicitly defining the Level 1 network group to exclude 0000 customers.

Counter Response

## It is acknowledged that there might be limited power flow impact from a Category 0000 customer on the rest of the network, however, additional factors that should be considered:

- Network Stability & Security of Supply: Distribution networks are designed to meet specific security of supply standards. If a 0000 customer's connection necessitates network upgrades or reinforcement to maintain compliance with these standards, there is a belief that these customers should contribute to the costs alongside others. High demand from a direct connection could strain the DNO's existing infrastructure, potentially requiring upgrades or modifications. A large 0000 customer could contribute to network congestion, potentially driving the need to upgrade DNO's GSP assets (e.g., switchboards, circuit breakers), which could lead to future reinforcement needs if new demand arises in the area. Reference is made to Ofgem’s DCP 139's decision letter, which cited scenarios with "meshed" networks where 0000 customers might draw power from multiple GSPs, making it difficult to assert they use no particular assets.

- Thermal Issue: Thermal issues following a new 0000 connection could lead to board upgrade or replacement costs. This is identified as an increasingly common issue due to the frequency of new direct GSP connections.

## The proposer initially asserted that 0000 customers were the only group of customers across EDCM and CDCM exposed to forward-looking signals in respect of downstream voltages. A workgroup member noted that this was not correct, citing customers connected directly to a BSP (with category 1100). Those customers are included in a network group which include both the BSP transformers (typically 132/33kV) and the downstream 33kV circuits. The proposer accepted this, but noted that the forward-looking signal for 0000 customers relates entirely to assets downstream of their connection (typically the 132kV circuits) while for 1100 customers the FCP charge relates to a mixture of upstream assets (typically the 132kV/33kV transformers) and downstream assets (typically the 33kV circuits).

Argument 2: FCP Charge 1 Capacity Calculation Process

Initial Counterargument

## A counter view to what has been set out in the Change Proposal is that a fundamental principle of the FCP methodology is its "zonal" nature, aiming to reflect predicted reinforcement costs within a network "group" rather than for individual assets. This is based on the premise that reinforcement within a zone could be triggered by any customer connected to that group, and all connected customers potentially benefit. Costs are shared across all customers in the group. According to the FCP methodology, GSP busbar assets are assigned to Level 1 network groups, and any customers connected to the busbar are also assigned to a Level 1 network group.

Proposer's Perspective

## The Proposer challenges the statement that reinforcement actions by any customer connected to a network group could trigger reinforcement and benefit all customers. It is argued that for 0000 customers, their actions cannot trigger thermal reinforcement on downstream Level 1 circuits (e.g., 132kV in E&W; 33kV in Scotland), nor can they benefit from such reinforcement.

Counter Response

## In response,  refers back to its earlier arguments regarding network stability, security of supply and thermal issues. It emphasises that it cannot be definitively stated that the wider network would not be impacted by the connection of 0000 customers, as not all 0000 connections are similar and their impact on wider networks can vary.

Argument 3: Network Use Factor Calculation Process

Initial Counterargument

## The proposer initially asserted that the statement in DCUSA that “Category 0000 Connectee are deemed not to use any network assets other than sole use assets" supports the rationale for the proposer. A workgroup member noted that this statement only applies to the calculation Network Use Factors (NUFs), not the calculation of FCP or LRIC Charge 1. The proposer accepted this, and noted that, in their view, other arguments for the change, such as the impact of 0000 customers on thermal reinforcement stand regardless of this point. The proposer also noted a clarificatory DCUSA change could be worthwhile.

Argument 4: Load Data and Reinforcement Cost Allocation

Initial Counterargument

## A counter view to what has been set out in the Change Proposal is that Load Data is a critical component in assessing future reinforcement costs for a network group (as shown in the Figure 1 of Schedule 17 – which provides a diagrammatic overview of the steps involved for import charges). Therefore, any demand growth contribution from connectees within a network group would contribute to the forecast of reinforcement costs for that group.

## An example is where a 0000 customer with significant demand capacity, if exempted from the 'Charge 1 capacity charge' (meaning exempted from the reinforcement costs attributable to their large demand capacity), would unfairly expose other customers at same Level 1 and also downstream customers at Level 2 and Level 3, who have much smaller demand capacities, to the full reinforcement cost.

Proposer's Perspective

## The Proposer disagrees with the view above, asserting that, under current arrangements, downstream customers are being subsidised by 0000 customers, and DCP 452 aims to remove this subsidy. It is argued that a 0000 customer's demand capacity does not contribute to the reinforcement of the 132kV circuits within the network group.

## The Proposer suggests that currently, a 0000 customer with a large capacity, artificially reduces the charge paid by downstream customers by picking up a portion of a reinforcement cost they do not drive. Using the example of £100k projected reinforcement cost and 100MW connected customers (charge of £1,000/MW), a new 50MW 0000 customer connecting would not increase the reinforcement cost but would increase total connected customers to 150MW, reducing the charge to £667/MW. This illustrates that the unchanged reinforcement cost is inappropriately socialised across more customers, despite the 0000 customer not driving that cost. To address this, changes to the definition of Cl and D in Paragraph 2.16 are proposed to exclude the capacity of 0000 customers from the denominator in the £/MW charge calculation.

Counter Response

## There is a view that the Proposer's statement that "currently, a 0000 customer with a large capacity artificially reduces the charge paid by downstream customers by picking up a portion of a reinforcement cost they do not drive.", is based on an assumption that connecting a 0000 customer has no impact on the wider network, which may not be the reality.

## It could be argued that if connecting a large demand 0000 customer burdens the network's remaining infrastructure, necessitating upgrades to comply with security of supply and network stability standards, it would increase the reinforcement cost of the entire network. This, in turn, would result in other customers at Level 1 and downstream customers at Levels 2 and 3 paying a higher Charge 1. Another scenario considered is that connecting a large demand customer could increase the chance of the network becoming congested, requiring DNO reinforcement to facilitate new connections in that area.

## There is an additional challenge made with respect to the Proposer's example, stating it is based on the assumption that the "£100k of projected reinforcement cost" remains unchanged following a new 0000 connection. It is argued that there are some scenarios where the new connection could lead to additional costs for the DNO's network, being:

- If a new 50MW 0000 customer connects to a 100MW network (resulting in a new total of 150MW load) and the DNO incurs an additional £50k cost to upgrade the network for the new large load, the Charge 1 would remain £1,000/MW. In this case, the forecasted additional reinforcement cost rise aligns with the total network's demand growth, keeping Charge 1 unchanged.

- If the additional cost is greater than £50k, the Charge 1 would be greater than £1,000/MW. This higher rate would then apply to all customers within that network, even if their individual loads are unchanged.

Question 3: Do you have any views on the counterarguments put forward with respect to the concepts of the FCP methodology and how they interact with the proposed approach of DCP 452?

#### Consideration of DCP 139 and Ofgem’s Decision to Reject

## In reviewing DCP 452, the Working Group considered the relevance of DCP 139: Non-Application of FCP charge for Category 0000 Customers which was raised in 2012 and ultimately rejected by Ofgem. While DCP 139 and DCP 452 share a similar objective, namely the removal of FCP locational charges for customers connected directly to a Grid Supply Point (GSP), the Working Group noted several important distinctions in context, rationale, and supporting evidence.

Overview of DCP 139

## DCP 139 was raised by British Gas and proposed that Category 0000 customers should be exempt from the FCP locational charge on the grounds that they do not use shared network assets and are therefore not cost drivers for reinforcement. The proposal argued that the application of the FCP charge to these customers was inconsistent with the treatment of network usage under the Network Use Factor (NUF) methodology, which assumes that 0000 customers use only sole-use assets.

Ofgem’s Decision and Rationale

## In its decision letter, Ofgem concluded that DCP 139 should not be implemented. The key reasons cited included:

- Ofgem noted that simply removing the FCP charge for 0000 customers did not guarantee a more cost-reflective outcome. In particular, it was unclear whether such customers might still contribute to reinforcement needs under certain network conditions (e.g. meshed networks or contingency scenarios).

- Ofgem emphasised that the FCP and NUF methodologies are based on different principles. Aligning them without broader consideration could risk undermining the internal consistency of the FCP approach.

- Ofgem suggested that 0000 customers might not be the only category of customer that could be deemed to not use any shared assets at a given network level and so consideration should have been given to those of customer categories.

Relevance to DCP 452

## The Working Group acknowledged the similarities between DCP 139 and DCP 452 but also identified several key differences that may support a different outcome in this case:

- DCP 452 is supported by detailed RFI data from DNOs and updated modelling undertaken by CEPA/TNEI, providing a clearer picture of the scale and distribution of the issue.

- The policy landscape surrounding the electricity industry has changed significantly since 2012, with greater emphasis meeting net zero and Clean Power 2030.The workgroup has identified that Ofgem’s statements on removing the FCP charge for all 0000 customers does not deliver a more cost-reflective outcome, namely for those customers at interconnected GSPs who can drive thermal reinforcement on downstream circuits and so should be exposed to a forward-looking charge in respect of those circuits. However, 0000 customers at non-interconnected can never drive thermal reinforcement on downstream 132kV circuits, hence DCP452 is expected to be more cost-reflective in all scenarios.

Question 4: Do you have any views on the relevance of DCP 139 and Ofgem’s decision to reject it? Please provide your views.

#### DCP 452 Proposed Solution

## The objective of the modification is to ensure that under the FCP variant of the EDCM, for Connectees connected directly to a GSP, identified by allocation of Customer Category 0000, are not incorrectly and non-cost-reflectively exposed to the locational charge/credit for the shared network in the Network Group associated with the network voltage downstream of the relevant GSP, as the Proposer asserts that such Connectees are deemed not to use any network assets other than Sole Use Assets.

## It should be noted that the proposed solution has been amended subsequent to the initial solution proposed within the original Change Proposal, although it does retain a portion of the original.

Question 5: Do you agree with the proposed solution for DCP 452?

#### DCP 452 Solution

## The intent is to remove Connectees with Customer Category 0000 which are connected to non-interconnected GSPs from the network group for that GSP, on the basis that they do not drive thermal reinforcement on the downstream circuits in that network group. It is believed that this will require amendments to Paragraph 2.7 in Schedule 17 and Paragraph 6.3 in Annex 1 of Schedule

Legal Text

#### DCP 452 Proposed Legal Text

## The DCUSA text to be implemented is in Attachment 2 and also below for ease of reference; and proposes to amend the following areas in Schedule 17:

- Paragraph 2.7 in Schedule 17, includes an additional sentence at the end of the current Paragraph which proposes to exclude, any Connectees with Customer Category 0000 that are directly connected to a GSP substation that does not operate in parallel with another GSP substation, from the relevant Network Group for that GSP substation.

- Aside from the original changes which amended the formulae for the import capacity charge, super-red import rate charge and super-red export rate credit (as set out in Paragraphs 6.2 and 6.3 in Schedule 17) such that those charges are set to zero for Connectees with Customer Category 0000, some additional context has also been added. The additional context includes a reference to Paragraph 2.7, to pick up that those Connectees also need to be connected to a GSP substation that does not operate in parallel with another GSP substation.

## DCP 452 proposes to modify Paragraph 2.7 of Schedule 17 as set out below:

Definition of Network Groups

2.7	The Authorised Network Model is split into Network Groups, thereby reflecting the zonal nature of the FCP model. A Network Group is a contained portion of the Authorised Network Model defined by physical, operational and technical boundaries that is not electrically connected to another part of the network at the same voltage level under normal operating conditions. A Network Group is defined as the network normally supplied from a Grid Supply Point (GSP) substation, a Bulk Supply Point (BSP) substation, or a Primary Substation. In situations where GSP substations, BSP substations or Primary Substations are operated in parallel, the network associated with such parallel GSP substations, BSP substations or Primary Substations is considered as one Network Group. In situations where GSP substations are not operated in parallel, Connectees which connect directly to those GSP substations (those which are assigned Customer Category 0000 according to Paragraph 15.6) shall be excluded from the relevant network group for that GSP substation.

2.8	Guidance relating to the definition of Network Groups is presented in section 6 (Network Groups) of Annex 1.

## DCP 452 proposes to modify Paragraph 6.1 to 6.3 of Schedule 17 as set out below:

6.1	Each tariff in the model is linked to one FCP location or network group.  Each FCP network group may be linked to a parent FCP network group and a grandparent FCP network group.  Each FCP network group may have a charge 1 in £/kVA/year associated with it.

6.2	The import charges for the application of charge 1 is given by the formulas:

For Connectees with Customer Category 0000 at GSP substations which are not operated in parallel (those excluded from the relevant Network Group as per Paragraph 2.7):

[p/kWh super-red rate] = 0

[p/kVA/day capacity charge] = 0

For other Connectees with zero average kW/kVA:

[p/kWh super-red rate] = ([parent charge 1 £/kVA/yr] * (abs[A1] / (SQRT(A1^2 + R1^2)) / [Super-red hours] *100) + ([grandparent charge 1 £/kVA/yr] * (abs[A2] / (SQRT(A2^2 + R2^2)) / [Super-red hours] *100)

[p/kVA/day capacity charge] = ([network charge 1 £/kVA/year] /[days in Charging Year]*100) + ([parent charge 1 £/kVA/yr]  * (–R1 * Average kVAr/kVA]) / (SQRT(A1^2 + R1^2)) / [days in Charging Year] *100) + ([grandparent charge 1 £/kVA/yr]  * (–R2 * [Average kVAr/kVA]) / (SQRT(A2^2 + R2^2)) / [days in Charging Year] *100)

For all other Connectees:

[p/kWh super-red rate] = [parent charge 1 £/kVA/yr] * (abs[A1] – (R1 * ([Average kVAr/kVA] / [Average kW/kVA])) / (SQRT(A1^2 + R1^2)) / [Super-red hours] *100 + ([grandparent charge 1 £/kVA/yr] * (abs[A2] – (R2 * ([Average kVAr/kVA] / [Average kW/kVA])) / (SQRT(A2^2 + R2^2)) / [Super-red hours] *100)

[p/kVA/day capacity charge] = [network group charge 1 £/kVA/year] / [days in Charging Year]*100

Where:

A1 and R1 are the values of the active power flow and reactive power flow modelled through the parent network group in the maximum demand scenario.

A2 and R2 are the values of the active power flow and reactive power flow modelled through the grandparent network group in the maximum demand scenario.

If both A1 and R1 are equal to zero, in respect of that network level in the formulas above, the term (abs[A1] / (SQRT(A1^2 + R1^2)) is set equal to 1,  (–R1 * Average kVAr/kVA]) / (SQRT(A1^2 + R1^2)) is set equal to zero, and ([Average kVAr/kVA] / [Average kW/kVA])) / (SQRT(A1^2 + R1^2)) is also set to zero.

If both A2 and R2 are equal to zero, in respect of that network level in the formulas above, the term (abs[A2] / (SQRT(A2^2 + R2^2)) is set equal to 1,  (–R2 * Average kVAr/kVA]) / (SQRT(A2^2 + R2^2)) is set equal to zero, and ([Average kVAr/kVA] / [Average kW/kVA])) / (SQRT(A2^2 + R2^2)) is also set to zero.

Any negative contributions to the [p/kVA/day capacity charge] or the [p/kWh super-red rate] from the parent or the grandparent network groups are set to zero.

Super red hours are the number of hours in the DNO Party’s super-red time band.

The average kW/kVA and average kVAr/kVA figures are forecasts for the Charging Year, based on data from the most recent regulatory year for which data were available in time for setting charges for the Charging Year. Specifically, active and reactive power consumptions are averaged over a super-red time band, which is a seasonal time of day period determined by the DNO Party to reflect the time of peak, and then divided by the Maximum Import Capacity (averaged over the same financial year).  If the DNO Party considers that the reactive consumption data relates to export rather than import (e.g. the average kVAr figure exceeds half of the Maximum Import Capacity) then the Maximum Import Capacity in the denominator should be replaced by the Maximum Export Capacity of the same Connectee. The average kVAr divided by kVA is restricted to be such that the combined active and reactive power flows cannot exceed the Maximum Import Capacity.

6.3	Charge 1 is applied to export charges as a credit.  The credit is expressed as a negative charge rate in p/kWh and is applied in respect of active power units exported during the DNO Party’s super-red time band. The credit rate is set to zero for Connectees who are assigned an F Factor of zero.  The credit rate is calculated as follows:

For Connectees with Customer Category 0000 at GSP substations which are not operated in parallel (those excluded from the relevant Network Group as per Paragraph 2.7):

[p/kWh super-red export rate] = 0

For all other Connectees:

[p/kWh super-red export rate] = -100*[Proportion eligible for charge 1 credits] *([network charge 1 £/kVA/year] + [parent charge 1 £/kVA/year] + [grandparent charge 1 £/kVA/year]) * ([Chargeable export capacity]/[Maximum export capacity]) /[number of hours in the super-red time band]

Where:

The proportion eligible for charge 1 credits is zero if the F factor that is assigned to the Connectee as described in the FCP methodology is equal to zero, and 1 otherwise.

The super-red generation rate is not applied to Connectees with zero Chargeable Export Capacity.

## DCP 452 proposes to modify Paragraph 6.1 to 6.4 of Annex 1 to Schedule 17 as set out below:

6.1	For the purpose of forecasting future reinforcement the network is broken down into a number of Network Groups.  The use of Network Groups for analysis is an important stage in assessing security of supply requirements given in ER P2/6. Network Groups are defined at hierarchical levels, each level being defined by the operating voltage of the source substations, such that separate Network Groups are defined for Primary Substation, BSP and GSP levels.

6.2	Each Network Group is a part of the Distribution System that consists of:

(a)	the transformation assets at a source substation; and

(b)	the network that:

(i)	operates at the same voltage as the lower voltage of these transformation assets; and

(ii)	is electrically connected to these transformation assets, under Normal Running Arrangements, excluding electrical connection through assets operating at voltages other than the lower voltage of the transformation assets.

6.3	The following exceptions apply:

(a)	where a source substation operates, under Normal Running Arrangements, with open point(s) on the lower voltage busbar such that there are separate sections of the busbar that are not electrically connected at the same voltage as the busbar, then these separate sections of busbar, and their associated network, shall be considered as separate Network Groups; and

(b)	where multiple source substations, with the same lower voltage of transformation assets, operate in parallel, under Normal Running Arrangements, through network operating at the same voltage as the lower voltage of the transformation assets, then these substations and their associated network shall be considered as a single Network Group; and

(c)	where GSP substations do not operate in parallel under Normal Running Arrangements, Connectees connected directly to those GSP substations (those which are assigned Customer Category 0000 according to Paragraph 15.6 of Schedule 17) shall be excluded from the relevant Network Group for that GSP substation.

6.4	Where a Network Group has, under Normal Running Arrangements:

(a)	no demand(load) or demand (generation) connected either within the Network Group, or any lower voltage Network Group associated with it; and

(b)	the Network Group exists solely for the purposes of providing security of supply support to an adjacent Network Group, through closure of open point(s) between such Network Groups,

(c)	then such Network Groups shall be considered as part of the adjacent Network Group to which they provide security of supply support (an example of such instances would be Network Groups that would otherwise be associated with transformers that operate on ‘hot standby’ under Normal Running Arrangements).

Text Commentary

## The proposed legal text changes:

- Include the addition of an exception (as set out in the modified Paragraphs 6.1 to 6.4 of Annex 1 to Schedule 17) to remove Connectees from the Level 1 Network Group if they are deemed to be in the Customer Category of 0000 and are connected to a GSP that does not operate in parallel under Normal Running Arrangements; and

- modifying the calculation of the FCP-element of the import capacity charge, super-red import rate charge and super-red export rate credit (as set out in Paragraphs 6.2 and 6.3 in Schedule 17) which results in those charges being set to zero for Connectees which meet the new exception above.

Question 6: Do you have any comments on the proposed legal text for DCP 452? If so, then please provide examples or supporting rationale.

Code Specific Matters

#### Reference Documents

## This issue was discussed at the Distribution Charging Methodologies Development Group (DCMDG), the presentation slides and discussion output of which can be found in the post meeting pack for meeting 84.

Relevant Objectives

#### Assessment Against the DCUSA Objectives

## For a DCUSA Change Proposal to be approved it must be demonstrated that it better meets the DCUSA Objectives. The Proposer of DCP 452 believes that the proposed solution will better facilitate DCUSA Charging Objectives two, three and six. The rationale for these decisions can be found in the paragraphs that follow the table of DCUSA Charging Objectives below.

| DCUSA Charging Objectives | Identified impact |
| --- | --- |
| 1 That compliance by each DNO Party with the Charging Methdologies facilitates the discharge by the DNO Party of the obligations imposed on it under the Act and by its Distribution Licence | None |
| 2 That compliance by each DNO Party with the Charging Methodologies facilitates competition in the generation and supply of electricity and will not restrict, distort, or prevent competition in the transmission or distribution of electricity or in participation in the operation of an Interconnector (as defined in the Distribtution Licences) | Positive |
| 3 That compliance by each DNO Party with the Charging Methodologies results in charges which, so far is reasonably practicable after taking account of implementation costs, reflect the costs incurred, or reasonably expected to be incurred, by the DNO Party in its Distribution Business | None |
| 4 That, so far is consistent with Clauses 3.2.1 to 3.2.3, the Charging Methodologies, so far as is reasonably practicable, properly take account of developments in each DNO Party’s Distribution Business | None |
| 5 That compliance by each DNO Party with the Charging Methodologies facilitates compliance with the Regulation on Cross-Border Exchange in Electricity and any relevant legally binding decisions of the European Commission and/or the Agency for the Co-operation of Energy Regulators. | None |
| 6 That compliance with the Charging Methodologies promoted efficency in its own implementation and administration | Positive |

## The Proposer’s rationale with respect to their position on the applicable DCUSA Charging Objectives is set out below:

- Charging Objective 2: this Change Proposal will ensure that customers directly connected to non-interconnected GSPs do not face forward-looking DUoS charges in respect of thermal reinforcement on downstream network assets which they can never drive. As a result, it will better facilitate competition in the generation of electricity between users connected at different voltage levels.

- Charging Objective 3: this Change Proposal will remove the application of forward-looking charges in respect of thermal reinforcement to customers directly connected to non-interconnected GSPs in respect of network assets on which they will never drive thermal reinforcement. The charges those users face will be more cost-reflective as a result.

- Charging Objective 6: this Change Proposal will remove a contradiction in DCUSA, promoting efficiency in its implementation.

Question 7: Which of the DCUSA Charging Objectives does DCP 452 better facilitate? Please provide supporting comments.

Impacts & Other Considerations

#### Model Amendments (EDCMs)

WORKING GROUP REQUEST FOR THE PRODUCTION OF AMENDED MODELS:

## The drafting amendments suggested by the Proposer were provided to the appointed modelling consultants on for the purposes of creating the set of models for the 2027/28 charging year.

CHANGES MADE TO THE MODELS TO ACCOMMODATE DCP 452:

## The appointed DCUSA modelling consultants created modified versions of the EDCM models (both the FCP and LRIC versions) in line with the draft legal text and with the aid of the model specification document provided by the Working Group. The updated models and a document that outlines the structural changes made to the models and other relevant information are located within Attachment 3 to this consultation.

## Section 3 within the document which outlines the modifications to the models that were necessary to implement DCP 452 explains that in both EDCMs (FCP and LRIC), changes were made in the following information sheets:

- ‘Version control’.

- ‘Cover’.

## It was noted that no changes were made to any of the input sheets.

## For both EDCMs (FCP and LRIC), changes were made in the following calculation sheets:

- ‘Charge 1 (LRIC). “Tariff information” –

  - In “Tariff information” row 21 has been added. The row is labelled under “Tariff Information” as “Customer category for demand scaling”.

- ‘Charge 1 (FCP). “Tariff information” –

  - In “Tariff information” row 21 has been added. The row is labelled under “Tariff Information” as “Customer category for demand scaling”.

  - In “Section 304-A: Identification of FCP parent and grandparent group” row 53 has been added. The row is labelled “Override FCP Charge 1 if connected directly to Grid Supply Point”.

  - In “Section 304-D: Super-red charge” the calculations were changed for the ‘Super-red rate’ for all tariffs.

  - In “Section 304-E: Capacity charge” the calculations were changed for the ‘Capacity charge’ for all tariffs.

  - In “Section 304-G: Export charge 1” the calculations were changed for the ‘Super-red export rate’ for all tariffs.

## DNOs, who are also Working Group members have successfully populated the DCP 452 EDCM (FCP) models and have confirmed that they have been able to replicate the expected outputs from the modified models.

#### Qualitative Customer Impact Assessment

Effects on customers in category 0000

## Customers in category 0000 no longer receive credits from the charge 1 super-red export rate. The export capacity and fixed charge are not impacted for these customers. As a result, the only change in export revenue is an increase in net revenue recovered from the super-red units for generation customers in category 0000.

## The import capacity charge for all 0000 customers decreases since these customers no longer receive a charge 1 capacity charge. However, there is an increase in the import fixed charge for final demand customers. The loss of revenue from the charge 1 capacity charge for 0000 customers leads to a shortfall in total import capacity charge revenue. This shortfall is compensated for via an increase in the residual added to the fixed charge for final demand sites. The impact on charges for generation and demand customers in category 0000 are shown in the table below.

| Charge | Import (final demand) | Import (non-final demand) | Export |
| --- | --- | --- | --- |
| Capacity charge | Decrease | Decrease | - |
| Super-red unit charge | - | - | Decrease in credits (set to zero) |
| Fixed charge | Increase | - | - |

Effects on other EHV customers (not in category 0000)

## DCP 452 has no impact on any of the three export charges (capacity charge, super-red and fixed) for other EHV customers. However, there is an indirect effect on import fixed charge for EHV final demand customers. As explained in the section above, the import fixed charge increases for final demand customers to recover the residual revenue shortfall. The effects on import charges experienced by EHV customers not in category 0000 is summarised in the table below.

| Charge | Import (final demand) | Import (non-final demand) | Export |
| --- | --- | --- | --- |
| Capacity charge | - | - | - |
| Super-red unit charge | - | - | - |
| Fixed charge | Increase | - | - |

Cumulative effects on EDCM revenue

## The reduction in revenue recovered from 0000 customers (via a decrease in import capacity charge) is compensated for by an increase in revenue recovered from EHV final demand customers (via an increased import fixed charge for these customers). However, there is a slight decrease in total revenue recovered from import charges in total. This is partially due to the rounding permitted by the EDCM and an indirect effect on import fixed charge revenue caused by the change in super-red export credits (no longer given to 0000 customers). In short, the increase in revenue from super-red units causes a small decrease in the EDCM demand revenue target, thus decreasing the residual added to the fixed charge for final demand sites. The resulting impact is a small decrease in total revenue recovered from import charges.

## The cumulative effect on total revenue recovered from export charges is an increase. The magnitude of this increase is dependent on the number of 0000 customers and the value of credits they no longer receive from the super-red export rate. The working group will need to use real data in the EDCM to determine the exact magnitude of increase in revenue recovered from export charges for each licence area.

## The cumulative effect on total revenue collected from the EDCM is an increase, again dependent on the number of 0000 customers and the value of credits they no longer receive from the super-red export rate. This is because the increase in revenue recovered from export charges will always exceed the decrease in revenue recovered from import charges. This is summarised in the table below.

| Type of revenue recovered | Impact |
| --- | --- |
| Export | Net increase |
| Import | Net decrease |
| Total | Net increase |

Effects on customers in CDCM and PCDM models

## There is no impact on the PCDM from DCP 452 since neither the EDCM outputs to the PCDM (notional EHV asset values) nor CDCM outputs to the PCDM (CDCM notional EHV asset values; Pass-through Smart Meter Communication Licence Costs) are affected.

## As explained previously, DCP 452 can only increase revenue recovered from the EDCM. Any increase in EDCM revenue will reduce CDCM target revenue via the “Revenue raised outside CDCM - EDCM and Certain Interconnector Revenue” input on the CDCM ‘General inputs’ sheet. As a result, DCP 452 may reduce CDCM customers’ residual charges (typically through the banded fixed charge, but sometimes through volumetric charges if the residual is sufficiently negative).

## Any decrease in CDCM charges may be small to the point of being negligible because: (i) EDCM revenue is small relative to CDCM revenue; and (ii) changes to EDCM revenue primarily come from 0000-connected export customers, which DNOs may not have many (or even any) of. The working group will need to use actual EDCM data to determine the exact magnitude of effects on CDCM charges.

#### Quantitative Customer Impact Assessment

## This section sets out the results of the RFI, received from all relevant DNOs, with the key findings including:

- Total number of impacted customers: 59

  - 12 customers receiving generation credits

  - 57 customers incurring demand charges

- Total number of EDCM customers across relevant DNOs: 1,532

- Aggregate financial impact:

  - Super-red credits: approximately £579,441

  - Capacity charges: approximately £1,371,159

## The below highlights the differences between individual DNOs on an anonymised basis:

- DNO 1: 47 customers impacted; ~£711k in capacity charges; ~£270k in super-red credits.

- DNO 2: 4 customers impacted; ~£405k in capacity charges; ~£310k in super-red credits.

- DNO 3: 7 customers impacted; ~£253k in capacity charges.

- DNO 4: 1 customer impacted; ~£1.3k in capacity charges.

- DNOs 5 and 6; no customers impacted.

## Following receipt of the updated EDCMs, those Working Group members who use the FCP variant of the EDCM fully test their models by using the same input data as for when they set 2026/27 tariffs at the end of 2024. The results of which were confirmed to be in line with the qualitative description provide by the modeling 	support consultants noted above.

## The Working Group acknowledged the value of this analysis and noted that the non-anonymised set of data could be provided to Ofgem by the Secretariat alongside the Change Declaration, and should Ofgem require more detailed evidence, customer-level data could be shared securely via the DCUSA Secretariat.

#### Impacts on other Significant Code Reviews (SCRs) or other significant industry change projects

## The potential interaction of DCP 452 with Ofgem’s DUoS SCR is detailed in paragraphs 4.4 to 4.8 of this consultation.

## It is not believed that this CP will impact on any other existing SCR but the Proposer noted the below industry projects.

Surplus residuals

## Surplus residuals have required derogations from publishing charges in line with the DCUSA. Some DNOs using FCP have held FCP charges at the level of previous years to avoid excessive volatility driven by surplus residuals. This Change Proposal would not directly interact with ongoing work on surplus residual issues, but may slightly reduce FCP-related revenues, so slightly reduce surplus residuals.

REMA

## REMA may fundamentally change the context of locational signals. Connectees may face locational operational signals under a zonal wholesale market, or face sharper locational signals through network charges (primarily TNUoS) under a reformed national market. It is very unclear how REMA will impact DUoS. Hence corrections such as this Change Proposal should continue to be made in the meantime.

Clean Power 2030

## The Clean Power 2030 Action Plan1 sets out the mix of technologies needed to achieve a decarbonised power system by 2030. Incorrectly applied locational signals could be a barrier to developing projects which are Clean Power 2030 aligned, so should be corrected.

#### Impacts on other Industry Codes

## The Working Group does not believe that DCP 452 has any impact on any other industry codes.

#### Environmental Impacts

## In accordance with DCUSA Clause 11.14.6, the Working Group assessed whether there would be a material impact on greenhouse gas emissions if DCP 452 were implemented. The Working Group did not identify any material impact on greenhouse gas emissions from the implementation of this CP.

#### Engagement with the Authority

## The Authority have been engaged with the development of this CP as an observer of the Working Group.

Question 8: Are you aware of any wider industry developments that may impact upon or be impacted by DCP 452?

Implementation

## The proposed implementation date is 1 April 2028, in line with the next publication of DUoS charges by DNOs. This would require an Authority decision by 01 October 2026 (at the very latest) to enable DNOs to incorporate the change in charge setting in late 2026. The Proposer considers that there is a strong case for derogations to apply this Change Proposal sooner in respect of heavily impacted new Connectees, which will be pursued outside of the DCUSA Change process.

Question 9: The proposed implementation date for DCP 452 is 01 April 2028. Do you agree with the proposed implementation date? If not, then please provide your rationale.

Consultation Questions

## The Working Group is seeking industry views on the following consultation questions:

| Number | Questions |
| --- | --- |
|  | Do you understand the intent of DCP 452? |

## Responses should be submitted using Attachment 1 to dcusa@electralink.co.uk no later than, DAY MONTH YEAR.

## Responses, or any part thereof, can be provided in confidence. Parties are asked to clearly indicate any parts of a response that are to be treated confidentially.

Attachments

Attachment 1 – DCP 452 Consultation Response Form

Attachment 2 – DCP 452 Legal Text (Schedule 17)

Attachment 3 – DCP 452 EDCM Modelling Documentation

Attachment 4 – DCP 452 Impact Assessment