DCP 460 Options

Overview

The change proposal identified two distinct but related issues:

- a postcode lottery for transmission-related connection costs in the different DNO areas

- unsustainable financial burden on individual customers (in those DNO areas which pass through all transmission-related connections costs)

The sections below consider options for these separately.

- Postcode lottery for connection costs (CUSC methodology issue for CMP460)

This arises from the Transmission charging regime where Infrastructure and non-Infrastructure sites, as categorised by the TOs in accordance with the CUSC charging methodology.

With the increase in Infrastructure sites, previously sole user transmission connection sites becoming shared by two or more transmission users, the split between the two categories is expected to be about a 40:60  (in NGED’s area), so the likelihood of different treatment arising from which GSP a connection is made to is high.

- Variance in DNO handling of transmission connection asset works related charges [current and future forms] (RIIO ED/DCUSA CCCM issue)

In the case of transmission sites that are not shared “connection sites” , the handling of transmission connection asset works ,are not treated in the same way across all DNOs.

.  this

There are a number of options to address this:

  - No T costs passed through to D connecting customers but recovered via DUoS

This would ensure full consistency irrespective of the classification of the GSP from the T charging.

  - No T costs passed through to D connecting customers (but recovered via DUoS) unless the GSP is to feed one customer

This would treat the new GSP as sole use and therefore extension assets and be fully chargeable to the connection customer at D.  This would retain consistency with D charging principles as the assets would be shared if more than one customer but sole if only the single customer.  This may avoid any gaming where a customer would avoid GSP costs if they were directly connected at T but option 1.1 was in place.

  - Extend the voltage rule to T charges and recover more via DUoS

An alternative is to extend the D “voltage rule” principle to T charges.  If this was applied then generation connecting at 132kV would pay for any 132kV costs at the new GSP; they would not pay for any of the transformer or higher voltage work.  Generation connecting at 33kV or below would not be exposed to any costs.

  - Application of a HCPT to limit recovery via DUoS

In all the examples above, consideration of whether a HCPT principle should apply would need to be considered.  For context, for a 5MW/MVA connection, the thresholds are £1m for generation and £8.6m for demand and for 50MW/MVA are £10m and £86m.

  - Implementation

Implementation of any of these options is likely to result only in a reduction in costs for connecting customers (or no change).  Therefore, there is the potential for this to be applied to the application of TMO4+ to the existing queue.  If this was the desired approach, then the charging methodology would need to be updated before DNOs issued revised connection offers to successful Gate 2 projects.

- Unsustainable financial burden on individual customers

This arises where DNOs do pass on the costs of any T changes to the connecting customers.  Where there are a number of customers, these costs are typically shared based on the respective capacities.  The issue arises if any customers fall away, either voluntarily withdrawing, being terminated or not meeting Gate 2 requirements.  In these situations, the totality of the costs falls to the “last man standing”.  This provides uncertainty and well as increase in costs which can happen at any time in the project lifecycle.

There are a number of options to address this:

  - Cost apportionment

A form of cost apportionment could be applied such that any connecting customers only pay a proportion of the new GSP.  This would have the effect of fixing the cost exposure for an individual connecting customer and remove the dependency of the actions of others.  The approach to cost apportionment could mirror the existing ones for distribution, however, their applicability would need consideration, particularly for fault level.  It should be noted that this could result in situations where the charge to the connecting customer is relatively small, for example if a 5MW generator triggered a new GSP, then the cost impact on DUoS customer would be very similar to those in scenario 1 above.

  - Cost apportionment with a threshold

A cost apportionment approach may benefit from an explicit lower threshold below which costs aren’t passed through to connecting customers.  For England and Wales this could be aligned to the thresholds for the requirement for a Transmission Evaluation Assessment but these vary by GSP at either 1MW or 5MW so would need consideration.

  - Application of the voltage rule to T charges

Consideration of the voltage rule would also be appropriate.  Currently a connecting customer does not pay for any reinforcement of the distribution system at the voltage level above its point of connection.  A generation connection at HV would therefore not pay for any reinforcement at 33kV or 132kV.  The logic of charging them for any transmission assets therefore needs consideration.

  - Application of a HCPT

In all the examples above, consideration of whether a HCPT principle should apply would need to be considered.  This could mitigate the issue identified in 2.1 but does add complexity.

  - Implementation

Implementation of any of these options is likely to result in increased costs for some connecting customers.  Therefore, the potential for this to be applied to the application of TMO4+ to the existing queue is more challenging.  If this was the desired approach, then implementation may need to be considered consistent with how Access SCR was done ie it applies to applications received after a specified date.  Customers would be able to cancel an existing contract and reapply but would be subject to the requirements of the new TMO4+ requirements.

Scenarios

To help illustrate how the different options could work, three scenarios are considered (these are based on England and Wales, similar principles will apply in Scotland and can be added for the consultation):

- Generation (60 MW) connecting at 132kV

- Generation (20 MW) connecting at 33kV or below

- Demand (80 MVA) connecting at any voltage

The diagrams above illustrate scenarios 1 & 2.

To help illustrate the options, the new GSP is assumed to create 240MW of capacity and cost £80m of which £5m is associated with the 132kV work on the transmission side of the interface.

|  | Option | Description | Charges to connecting customer | Costs to all DUoS customers | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.1 | No T costs passed through to individual D connectees | Simple option whereby all T costs recovered from all DUoS customers. | £0 | £80m | Gives complete consistency as all customers pay charges for T work irrespective of current or future classification of GSP. |
| 1.2 | No T costs passed through to individual D connectee unless the GSP is to feed one customer | This is as per 1.1 but with an exception if a new GSP is created “wholly or mainly” for a single customer. It is not meant to apply to where a new customer triggers a GSP and the extra capacity could be used by others. | £80m | £0 | Issues that would need resolving: - Defining when it would apply eg what % of the substation is used - If there is any residual capacity, does the fist comer get any rebate? (simplest approach would be no, consistent with ECCR principles) |
| 1.3 | Extend the voltage rule to T charges | This is an alternative approach which gives different outcomes depending on the type and POC of the D customer. This means that customers only contribute to work at the same voltage level as the POC plus the voltage level above. | - £5m - £0 - £0 | - £75m - £80m - £80m | This provides a consistent approach of distribution charging principles across T/D boundary. It largely achieves consistency in terms of charging with the anomaly limited to generation connecting at 132kV but with reduced levels of charging. |
| 1.4 | Application of a ‘High Cost Project Threshold’ (HCPT) | This is an additional feature which could be added to any of the options above. Assuming consistency with the existing HCPT (that applies only to D reinforcement), if the applicable costs are greater than the threshold, then the customer is charged all the applicable reinforcement. Only the reinforcement at the same voltage level plus the one above is considered. Figures assume it is triggered. | £80m if POC is 132kV £5m if POC is 33kV £0m if POC is HV £80m if demand | £0 if POC is 132kV £75m if POC is 33kV £80m if POC is HV £0 if demand | Some sort of HCPT could provide some degree of protection for DUoS customers for extreme cases eg a small new connection triggers a new GSP. But this does result in an ambiguity as it would only apply to situations where the GSP is non-Infrastructure. Consideration of how the threshold is calculated would need consideration as the existing thresholds were calculated based on distribution costs and therefore may not be applicable for T costs. |
|  | Option | Description | Charges to connecting customer | Costs to DUoS customers | Comments |
| 2.1 | Cost apportionment | The approach would be for the connecting customer to pay their proportion of the total capacity created. | - £20m - £6.7m - £26.7m | - £60m - £73.3m - £53.3m | This is fairly simple for thermal capacity but may need further consideration if fault level is the trigger for the new GSP, particularly the appropriateness of demand paying towards the costs. The issue is how far down the network charges are leveed. In theory this could result in charges(albeit small) being passed to a domestic customer that ends up connecting to a new GSP. |
| 2.2 | Cost apportionment with a threshold | This would be similar to 2.1 but adding an explicit lower threshold. The threshold could be defined by the size of the new connection eg 5MW or by the voltage level of the POC. The costs in the table have assumed a threshold of 5WM. | - £20m - £6.7m - £26.7m | - £60m - £73.3m - £53.3m | This potentially resolves the issue of how far down the network the costs are leveed. The choice of threshold will need consideration as in some cases a 1MW new connection could go through the transmission evaluation. |
| 2.3 | Application of the voltage rule to T charges | This means that customers only contribute to work at the same voltage level as the POC plus the voltage level above. Cost apportionment would then be applied to the work in scope. | - £20m - £0.4m - £26.7m | - £60m - £79.6m - £53.3m | This has no effect on connections at 132kV but does at 33kV and below. |
| 2.4 | Application of a ‘High Cost Project Threshold’ (HCPT) | This is an additional feature which could be added to any of the options above. Assuming consistency with the existing HCPT (that applies only to D reinforcement), if the applicable costs are greater than the threshold, then the customer is charged all the applicable reinforcement. Only the reinforcement at the same voltage level plus the one above is considered. Figures assume it is triggered. | £80m if POC is 132kV £5m if POC is 33kV £0m if POC is HV £80m if demand | £0 if POC is 132kV £75m if POC is 33kV £80m if POC is HV £0 if demand | Some sort of HCPT could provide some degree of protection for DUoS customers for extreme cases eg a small new connection triggers a new GSP. But this does result in an ambiguity as it would only apply to situations where the GSP is non-Infrastructure. Consideration of how the threshold is calculated would need consideration as the existing thresholds were calculated based on distribution costs and therefore may not be applicable for T costs. |