SCHEDULE 17: EHV CHARGING METHODOLOGY (FCP MODEL)

18. DEMAND SCALING

18.22	Where excessive surplus residual revenue occurs within the EDCM model at a tariff set case (demonstrated by negative fixed charges or where the load forecast applied for the charging year causes the FCP models not to converge), the following intervention option(s) should be applied by affected DNOs:

- Where the principal cause of excessive surplus residual is a reduction in allowed revenue, reapportion negative fixed charges for final demand consumers within a residual band to the capacity charge for the same group of consumers.

- Where the principal cause of excessive surplus residual is an increase in forward-looking charges,  the DNO may:

- Carry over locational components and network use factors from the previous year; or

- Apply the load growth assumptions used to create prices in the previous year’s tariff setting process together with the latest Authorised Network Model.

18.23	Where an intervention option is utilised from paragraph 18.22 and does not completely eradicate credit fixed charge rates due to the scale of surplus residual revenue, a second intervention option from those listed may also be applied.

18.24	DNOs should stipulate within the DUoS Charging Statements which intervention option(s) per 18.22, if any, have been applied for the tariff setting year.

22. CHARGES FOR NEW CONNECTEES

22.5	If an intervention option to alleviate excessive surplus residual has been applied at a tariff set case in line with section 18.22, any new EDCM connectees for that year should have their charges calculated on an equivalent basis.

Annex 1 - Implementation Guide

5. NETWORK DEMAND DATA

5.4	Per Paragraph 18.22, if excessive surplus residual revenue occurs within the EDCM model and this is driven by the demand (load/s) forecast within the LTDS, the DNO may apply the load growth scenario from the previous year’s tariff setting process to the latest Authorised Network Model to eliminate negative fixed charges within the EDCM model.